| Daimler Truck AG | Scania AB | IVECO Group |
| --- | --- | --- |
| Daimler Buses GmbH | Scania CV | DAF Trucks N.V. |
| Volvo Truck Corporation | Volvo Bus Corporation | VDL Bus & Coach B.V. |
| MAN Truck & Bus SE | Renault Trucks | Ford Otosan |

FMS-Standard description Version 05

11.11.2025

© HDEI / BCEI Task Force

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE | Scania AB Scania CV Volvo Bus Corporation Renault Trucks | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan | Name of document | FMS-Standard |  | Page | 2 (74) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) | ACEA Task Force HDEI/BCEI |  | Date 11.11.2025 | Approved final | Issue Version 05 | Reg. no. |  |
| Subject |  | FMS-Standard description |  |  |  |  |  |
| General annotations - Data might be not available during ignition off / main switch off / charging * - Most of the values are reliable after ca. 10 seconds after “ignition on”. * - Physical Layer according to ISO 11898 (250kb/s) - Application Layer according to SAE J1939/ 71 (J1939DA_202007) - Data Link Layer according to SAE J1939/ 21 - If there is a discrepancy between definitions in this document and the SAE, the SAE documents are valid only except broadcast for * PGN 0x00FE6B (Driver ID) * PGN 0x00FE70 (Combination Vehicle Weight) * PGN 0x00FEE6 (Time/Date) * PGN 0x00FCB7 (Vehicle Electrical Power #4) * PGN 0x00FDC2 (Electronic Engine Controller 14) * PGN 0x00FEAF (Fuel Consumption (Gaseous)) * PGN 0x00F009 (Vehicle Dynamic Stability Control 2) * PGN 0x00FEFC (Dash Display 1): Only used if different fuel types in different storage container Fuel Level 1 represents always the fuel used for driving without indication of the location of the storage container. Fuel Level 2 is not indicating the location of the storage container * PGN 0x00FC5E (High Voltage Energy Storage System History) This PGN is not sent as a BAM * PGN 0x00F009 (Vehicle Dynamic Stability Control 2) repetition rate is 100 ms instead of 10 ms - The priority/source address of each OEM is different. - If the information is delivered the function/data has to be delivered according to FMS-standard definition. |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE | Scania AB Scania CV Volvo Bus Corporation Renault Trucks | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan | Name of document | FMS-Standard |  | Page | 3 (74) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) | ACEA Task Force HDEI/BCEI |  | Date 11.11.2025 | Approved final | Issue Version 05 | Reg. no. |  |
| Subject |  | FMS-Standard description |  |  |  |  |  |
| - The description of the connector(s) can be downloaded in the download area of FMS-Standard (Bus and Truck) - If the information is not available, the function/data has to be sent as not available according to SAE - “not used for (Bus) FMS-standard” means that there might be data sent according SAE but are not used in (Bus) FMS-standard interface. If no information is sent, then it has to be sent as “not available” (don’t care). - “reserved for (Bus) FMS-standard” means that as long as there is no definition it is sent “FF (not available)” - The accuracy/interpretation of signals might differ depending on vehicle brand/models *. - (Bus) FMS-Standard is designed for Diesel engines. If it is used in vehicles with other engine types (e.g., dual engine) the information delivered might be different: * - Some data are configuration relevant * * Details can be obtained from the OEM |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE | Scania AB Scania CV Volvo Bus Corporation Renault Trucks | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  | Name of document | FMS-Standard |  | Page 4 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) | ACEA Task Force HDEI/BCEI |  |  | Date 11.11.2025 | Approved final | Issue Version 05 | Reg. no. |  |
| Subject |  | FMS-Standard description |  |  |  |  |  |  |
| History |  |  |  |  |  |  |  |  |
| Truck |  |  | Bus |  |  |  |  |  |
| Changes / Addition to Vers. 01.00 Oct. 2009 - added history - change of DaimlerChrysler to Daimler - added Renault Trucks - update General Annotation - added description acc. SAE (based on Jan 2008 version) - deleted SAE ref as no longer valid - added additional comments - correction of PGNs (dez) in Example for BAM - added Priority to Example for BAM - added 2.2 Example SW Identification for buses and/or trucks - added Overview Messages |  |  | Changes / Addition to Vers. 00.01: - added Driver Identification DI - added Fuel Economy LFE - added Tell Tale Status TTS - added Example Tell Tale Status - added Overview Messages - added description acc. SAE J 1939 to the PGN´s - added additional comments to the PGN´s |  |  |  |  |  |
| Changes / Addition to Vers. 02.00 Sept. 2010 - update History - added 1.14 Ambient Conditions: AMB - added 1.15 Driver’s Identification: DI - added 1.16 Fuel Economy: LFE - added 1.2 EEC2: Engine Percent Load At Current Speed - added 1.17 PTO Drive Engagement: PTODE - added 1.18 High Resolution Fuel Consumption (Liquid): HRLFC - update 3. Overview Messages - ​ |  |  | Changes / Addition to Vers. 00.02 dated 07.07.2009 - Page 36: Remark for accelerator position: “Daimler calculate from torque demand” deleted |  |  |  |  |  |
| Changes / Additions to Vers. 02.00 Nov. 2010 - some editorial corrections |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  | Name of document FMS-Standard |  |  | Page 5 (74) |  |
| --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  | Date 11.11.2025 | Approved final | Issue Version 05 | Reg. no. |  |
| Subject FMS-Standard description |  |  |  |  |  |  |
| Version 03 dated 12.03.2012 One document for Bus and Truck FMS-Standard Truck and Bus: - update History - update company names - update General Annotation - added SPN 513 Actual Engine – Percent Torque - added High Resolution Fuel Consumption (Liquid) HRLFC - added Aftertreatment 1 Diesel Exhaust Fluid Tank 1 Level - added FMS Tell Tale Status : FMS1 - update example tell tale status - additions to additional comments - update Overview messages Truck section: - added Combination Vehicle Weight: CVW - added Electronic Retarder Controller 1: ERC1 - added 1.22 Air Supply Pressure : AIR1 - update 3. Overview Messages | Version 03 dated 12.03.2012 One document for Bus and Truck FMS-Standard Truck and Bus: - update History - update company names - update General Annotation - added SPN 513 Actual Engine – Percent Torque - added High Resolution Fuel Consumption (Liquid) HRLFC - added Aftertreatment 1 Diesel Exhaust Fluid Tank 1 Level - added FMS Tell Tale Status : FMS1 - update example tell tale status - additions to additional comments - update Overview messages Bus section: - connector description in separate file |  |  |  |  |  |
| Version 03 dated 28.01.2016 Truck and Bus: - update History - correction of ISO No. for Tell Tale ID 53 | Version 03 dated 28.01.2016 Truck and Bus: - Update History - correction of ISO No. for Tell Tale ID 53 |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE | Scania AB Scania CV Volvo Bus Corporation Renault Trucks | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan | Name of document | FMS-Standard |  | Page | 6 (74) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) | ACEA Task Force HDEI/BCEI |  | Date 11.11.2025 | Approved final | Issue Version 05 | Reg. no. |  |
| Subject |  | FMS-Standard description |  |  |  |  |  |

| Version 04 dated 13.10.2017 Truck and Bus: - update History - added Electronic Brake Controller 1: EBC1 - added Electronic Engine Controller 14: EEC14 - added Fuel Consumption (Gaseous): GFC - added SPN 38 Fuel Level 2 - added Truck and Bus: Retarder Information : ERC1 (moved from Truck only) - added Tell Tale No 61-64 - update 3. Overview Messages - renamed PGN 0x00FEFC: Dash Display 1: DD1 Truck section: - added SPN 70 Parking Brake Switch - added SPN 527 Cruise Control States - renamed PGN 0x00FEF1: Cruise Control/Vehicle Speed 1: CCVS1 | Version 04 dated 13.10.2017 Truck and Bus: - update History - added Electronic Brake Controller 1: EBC1 - added Electronic Engine Controller 14: EEC14 - added Fuel Consumption (Gaseous): GFC - added SPN 38 Fuel Level 2 - added Truck and Bus: Retarder Information : ERC1 (moved from Truck only) - added Tell Tale no 61-64 - update 3. Overview Messages - renamed PGN 0x00FEFC: Dash Display 1: DD1 Bus section: - added SPN 527 Cruise Control States - added SPN5464 Hybrid Battery Pack Remaining Charge - added SPN 1807 Steering Wheel Angle |
| --- | --- |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  | Name of document FMS-Standard |  |  | Page 7 (74) |  |
| --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  | Date 11.11.2025 | Approved final | Issue Version 05 | Reg. no. |  |
| Subject FMS-Standard description |  |  |  |  |  |  |
| Version 04 dated 17.09.2021 Truck and Bus: - update History - added Tell Tale No 65-69 | Version 05 dated 24.11.2023 Truck and Bus: - update History - added Vehicle Weight: VW (moved from Truck only) - added SPN 13171 DC Charging State - added SPN 15268 HVESS Estimated Remaining Distance - added SPN 241 Tire Pressure - added SPN 929 Tire Location - added SPN 8428 Fuel Supply Estimated Remaining Distance - added SPN 97 Water In Fuel Indicator 1 - Update SPN 5837 Fuel Type list Bus section: - added SPN 1810 Longitudinal Acceleration - added SPN 1809 Lateral Acceleration - added SPN 8235 HVESS Thermal Management System Mode - added SPN 7315 Propulsion System Active - added SPN 175 Engine Oil Temperature 1 - added SPN 100 Engine Oil Pressure 1 - added SPN 170 Cab Interior Temperature - added SPN 177 Transmission Oil Temperature 1 - added SPN 8211 HVESS Total Energy Throughput |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE | Scania AB Scania CV Volvo Bus Corporation Renault Trucks | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan | Name of document | FMS-Standard |  | Page | 8 (74) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) | ACEA Task Force HDEI/BCEI |  | Date 11.11.2025 | Approved final | Issue Version 05 | Reg. no. |  |
| Subject |  | FMS-Standard description |  |  |  |  |  |

| Version 05 dated 07.07.2024 Truck and Bus: - update History - update General annotations - additional Annotations PGN FD7D FMS1 - additional comment PGN FAB8 EVSE1DCS1 Bus section - error correction PGN 64606 HVESSHIST - additional comment PGN 64606 | Version 05 dated 11.11.2025 Truck and Bus: - update History - update General annotations - added Tell Tale No 70-75 - additional comment for Tell Tales Bus section - ​ |
| --- | --- |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE | Scania AB Scania CV Volvo Bus Corporation Renault Trucks | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan | Name of document | FMS-Standard |  | Page | 9 (74) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) | ACEA Task Force HDEI/BCEI |  | Date 11.11.2025 | Approved final | Issue Version 05 | Reg. no. |  |
| Subject |  | FMS-Standard description |  |  |  |  |  |
| - PARAMETERS FOR FMS GATEWAY (ACCORDING TO SAE J1939) 11 - Parameters for Bus and Truck FMS-Standard 11 - Fuel Consumption: LFC 11 - Dash Display 1: DD1 12 - Electronic Engine Controller #1: EEC1 13 - Engine Hours, Revolutions: HOURS 14 - Vehicle Identification: VI 15 - FMS-standard Interface Identity / Capabilities: FMS 16 - High Resolution Vehicle Distance: VDHR 18 - Tachograph : TCO1 19 - Engine Temperature 1: ET1 21 - Ambient Conditions: AMB 22 - Driver's Identification: DI 23 - Fuel Economy: LFE 24 - Air Supply Pressure : AIR1 25 - High Resolution Fuel Consumption (Liquid): HRLFC 26 - Aftertreatment 1 Diesel Exhaust Fluid Tank 1 Information: AT1T1I 27 - FMS Tell Tale Status: FMS1 28 - Electronic Brake Controller 1: EBC1 32 - Electronic Engine Controller 14: EEC14 33 - Fuel Consumption (Gaseous): GFC 36 - Electronic Retarder Controller 1: ERC1 37 - Vehicle Weight: VW 38 - EVSE 1 DC Status 1 : EVSE1DCS1 39 - EV Operator Indicators 1 : EVOI1 40 - Tire Condition Message 1 : TIRE1 41 - Operator Indicators : OI 42 - Parameters for Truck FMS-Standard 43 - Cruise Control/Vehicle Speed 1: CCVS1 43 - Electronic Engine Controller #2: EEC2 45 - Service Information: SERV 46 - PTO Drive Engagement: PTODE 47 - Combination Vehicle Weight: CVW 48 |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE | Scania AB Scania CV Volvo Bus Corporation Renault Trucks | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan | Name of document | FMS-Standard |  | Page | 10 (74) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) | ACEA Task Force HDEI/BCEI |  | Date 11.11.2025 | Approved final | Issue Version 05 | Reg. no. |  |
| Subject |  | FMS-Standard description |  |  |  |  |  |
| - Parameters for Bus FMS-Standard 49 - Cruise Control/Vehicle Speed: CCVS 49 - Electronic Engine Controller #2 : EEC2 51 - Door Control 1: DC1 52 - Door Control 2: DC2 53 - Time / Date : TD 54 - Alternator Speed : AS 55 - Electronic Transmission Controller 2 : ETC2 56 - Air Suspension Control 4 : ASC4 57 - Vehicle Electrical Power #4 : VEP4 58 - Vehicle Dynamic Stability Control 2 : VDC2 59 - Engine Temperature 1: ET1 60 - Ambient Conditions: AMB 61 - High Voltage Energy Storage System Thermal Management System Status 1 : HVESSTS1 62 - High Voltage Energy Storage System History : HVESSHIST 63 - Hybrid System Status 1 : HSS1 64 - Engine Fluid Level/Pressure 1 : EFL/P1 66 - Transmission Fluids 1 : TRF1 67 - EXAMPLES 68 - Broadcast Announce Message (BAM) for Vehicle ID longer than 8 Byte 68 - Example SW Identification for buses and/or trucks 70 - Example FMS Tell Tale Status 71 - OVERVIEW MESSAGES 73 |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Page 11 (74) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  | ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  |  |  |  |  |  | Approved final |  |  |  |  |  |  |  | Issue Version 05 |  |  |  |  |  |  |  |  |  | Reg. no. |  |  |  |  |  |
| Subject |  |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Parameters for FMS gateway (according to SAE J1939) always MSB (Most Significant BIT) First - Parameters for Bus and Truck FMS-Standard - Fuel Consumption: LFC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FEE9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |  |
|  | 65,257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |  |
|  | 1000 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |  |
|  | Data Byte 1 | Data Byte 2 |  | Data Byte 3 | Data Byte 4 |  | Data Byte 5 |  |  |  |  |  |  |  | Data Byte 6 |  |  |  |  |  |  |  | Data Byte 7 |  |  |  |  |  |  |  |  |  | Data Byte 8 |  |  |  |  |  |  |  |  | Byte No |  |
|  |  |  |  |  |  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 |  | 3 | 2 |  | 1 | 8 | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 | Bit No |  |
|  | Not used for (Bus) FMS-Standard | Not used for (Bus) FMS-Standard |  | Not used for (Bus) FMS-Standard | Not used for (Bus) FMS-Standard |  | Engine total fuel used 0,5 L / Bit gain 0 L offset SPN 250 |  |  |  |  |  |  |  | Engine total fuel used 0,5 L / Bit gain 0 L offset SPN 250 |  |  |  |  |  |  |  | Engine total fuel used 0,5 L / Bit gain 0 L offset SPN 250 |  |  |  |  |  |  |  |  |  | Engine total fuel used 0,5 L / Bit gain 0 L offset SPN 250 |  |  |  |  |  |  |  |  | Name values values values SPN |  |
| Description acc. SAE J 1939: Total Fuel Used: Accumulated amount of fuel used during vehicle operation. Additional comment: Calculated values given as indications, not as contractual values. Might be set to “not available” if the High Resolution Fuel Consumption is available The mentioned resolution is not related to the accuracy of the signal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  |  |  |  |  |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  | Name of document FMS-Standard |  |  |  |  |  |  |  |  |  |  |  |  |  | Page 12 (74) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  |  |  |  |  | ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  |  |  |  |  | Issue Version 05 |  |  |  |  |  | Reg. no. |  |  |
| Subject |  |  |  |  |  |  |  |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1.2 Dash Display 1: DD1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FEFC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |  |
|  | 65,276 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |  |
|  | 1000 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |  |
|  | Data Byte 1 | Data Byte 2 |  |  |  |  |  |  |  | Data Byte 3 | Data Byte 4 |  | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  |  |  |  |  |  |  |  |  | Data Byte 8 |  | Byte No |  |
|  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  | 8 | 7 | 6 | 5 | 4 |  | 3 | 2 |  | 1 |  |  | Bit No |  |
|  | Not used for (Bus) FMS-Standard | Fuel Level 1 0,4 % / Bit gain 0 % offset SPN 96 |  |  |  |  |  |  |  | Not used for (Bus) FMS-Standard | Not used for (Bus) FMS-Standard |  | Not used for (Bus) FMS-Standard |  | Not used for (Bus) FMS-Standard |  | Fuel Level 2 0,4 % / Bit gain 0 % offset SPN 38 |  |  |  |  |  |  |  |  |  | Not used for (Bus) FMS-Standard |  | Name values values values SPN |  |
| Description acc. SAE J 1939: Fuel Level: Ratio of volume of fuel to the total volume of fuel storage container. When Fuel Level 2 (SPN 38) is not used, Fuel Level 1 (SPN 96) represents the total fuel in all fuel storage containers. When Fuel Level 2 is used, Fuel Level 1 represents the fuel level in the primary or left-side fuel storage container. Additional comment: Fuel Level 2 is only used if different fuel types in different storage container Fuel Level 1 represents always the fuel used for driving without indication of the location of the storage container. Fuel Level 2 is not indicating the location of the storage container The mentioned resolution is not related to the accuracy of the signal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  |  |  |  |  |  |  |  |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  |  |  |  |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  |  |  |  | Page 13 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  |  | ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  |  |  | Approved final |  | Issue Version 05 |  |  | Reg. no. |  |
| Subject |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1.3 Electronic Engine Controller #1: EEC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00F004 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 61,444 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 20 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 | Data Byte 2 |  | Data Byte 3 |  |  |  |  |  |  |  | Data Byte 4 |  |  |  |  |  |  |  | Data Byte 5 |  |  |  |  |  |  |  |  | Data Byte 6 |  | Data Byte 7 |  |  | Data Byte 8 |  | Byte No |
|  |  |  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 |  | 2 | 1 |  |  |  |  |  |  |  | Bit No |
|  | Not used for (Bus) FMS-Standard | Not used for (Bus) FMS-Standard |  | Actual Engine – Percent Torque 1 % / Bit -125 % offset SPN 513 |  |  |  |  |  |  |  | Engine speed 0.125 rpm / Bit gain 0 rpm offset SPN 190 |  |  |  |  |  |  |  | Engine speed 0.125 rpm / Bit gain 0 rpm offset SPN 190 |  |  |  |  |  |  |  |  | Not used for (Bus) FMS-Standard |  | Not used for (Bus) FMS-Standard |  |  | Not used for (Bus) FMS-Standard |  | Name Name values values values SPN |
| Description acc. SAE J 1939: Actual Engine – Percent Torque: The calculated output torque of the engine. The data is transmitted in indicated torque as a percent of reference engine torque (see the engine configuration message, PGN 65251). The engine percent torque value will not be less than zero and it includes the torque developed in the cylinders required to overcome friction. Engine Speed: Actual engine speed which is calculated over a minimum crankshaft angle of 720 degrees divided by the number of cylinders. Additional comment: The mentioned resolution is not related to the accuracy of the signal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  |  |  |  |  |  |  |  |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  |  |  |  |  |  |  |  |  |  |  |  |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  |  | Page 14 (74) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  |  |  |  |  |  |  |  |  |  |  |  | ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  |  | Reg. no. |  |  |
| Subject |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1.4 Engine Hours, Revolutions: HOURS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FEE5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |  |
|  | 65,253 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |  |
|  | 1000 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |  |
|  | Data Byte 1 |  |  |  |  |  |  |  | Data Byte 2 |  |  |  |  |  |  |  | Data Byte 3 |  |  |  |  |  |  |  | Data Byte 4 |  |  |  |  |  |  |  | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  |  | Data Byte 8 |  | Byte No |  |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  |  |  | Bit No |  |
|  | Engine total hours of Operation 0.05 h / Bit gain 0 h offset SPN 247 |  |  |  |  |  |  |  | Engine total hours of Operation 0.05 h / Bit gain 0 h offset SPN 247 |  |  |  |  |  |  |  | Engine total hours of Operation 0.05 h / Bit gain 0 h offset SPN 247 |  |  |  |  |  |  |  | Engine total hours of Operation 0.05 h / Bit gain 0 h offset SPN 247 |  |  |  |  |  |  |  | Not used for (Bus) FMS-Standard |  | Not used for (Bus) FMS-Standard |  | Not used for (Bus) FMS-Standard |  |  | Not used for (Bus) FMS-Standard |  | Name values values values SPN |  |
| Description acc. SAE J 1939: Engine total hours of Operation: Accumulated time of operation of engine. Additional comment: The mentioned resolution is not related to the accuracy of the signal Counter is Engine running dependant |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  | Name of document FMS-Standard |  |  |  |  |  |  | Page 15 (74) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  | ACEA Task Force HDEI/BCEI |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  |  | Reg. no. |  |  |
| Subject |  |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |
| 1.1.5 Vehicle Identification: VI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FEEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |  |
|  | 65,260 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |  |
|  | 10.000 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |  |
|  | Variable 1-n | Variable 1-n |  | Variable 1-n | Variable 1-n |  | Variable 1-n |  | Variable 1-n |  | Variable 1-n |  |  | Variable 1-n |  | Byte No |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Bit No |  |
|  | Vehicle identification number ASCII up to 200 characters * = Delimiter SPN 237 | Vehicle identification number ASCII up to 200 characters * = Delimiter SPN 237 |  | Vehicle identification number ASCII up to 200 characters * = Delimiter SPN 237 | Vehicle identification number ASCII up to 200 characters * = Delimiter SPN 237 |  | Vehicle identification number ASCII up to 200 characters * = Delimiter SPN 237 |  | Vehicle identification number ASCII up to 200 characters * = Delimiter SPN 237 |  | Vehicle identification number ASCII up to 200 characters * = Delimiter SPN 237 |  |  | Vehicle identification number ASCII up to 200 characters * = Delimiter SPN 237 |  | Name values values values SPN |  |
| Description acc. SAE J 1939: Vehicle identification number: Vehicle Identification Number (VIN) as assigned by the vehicle manufacturer. NOTE The ASCII character "*" is reserved as a delimiter. Annotations: - If the Vehicle ID is up to 8 Bytes (including) then it is broadcasted with PGN 00FEEC containing the vehicle ID and filled with “FF” at the unused bytes. - If the Vehicle ID contains more than 8 Bytes then a TP.CM (PGN 00EC00) with a minimum of two TP.DT (PGN 00EB00) will be used. Information might be a sub-set of the full VIN. ie: only last 8 bytes are sent see example 2.1 Additional comment: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  |  |  |  |  |  |  |  |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  |  |  |  |  |  |  |  |  |  |  |  |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  |  |  |  |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  |  |  | Page 16 (74) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  |  |  |  |  |  |  |  |  |  |  |  | ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  |  | Approved final |  | Issue Version 05 |  |  | Reg. no. |  |  |
| Subject |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1.6 FMS-standard Interface Identity / Capabilities: FMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FDD1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |  |
|  | 64,977 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |  |
|  | 10.000 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |  |
|  | Data Byte 1 |  |  |  |  |  |  |  | Data Byte 2 |  |  |  |  |  |  |  | Data Byte 3 |  |  |  |  |  |  |  | Data Byte 4 |  |  |  |  |  |  |  | Data Byte 5 |  |  |  |  |  |  |  | Data Byte 6 |  | Data Byte 7 |  |  | Data Byte 8 |  | Byte No |  |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  | Bit No |  |
|  |  |  |  |  |  |  |  |  | SW-version supported |  |  |  |  |  |  |  | SW-version supported |  |  |  |  |  |  |  | SW-version supported |  |  |  |  |  |  |  | SW-version supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name |  |
|  |  |  |  |  |  |  |  |  | Version number in |  |  |  |  |  |  |  | Version number in |  |  |  |  |  |  |  | Version number in |  |  |  |  |  |  |  | Version number in |  |  |  |  |  |  |  |  |  |  |  |  |  |  | values |  |
|  | Reserved for (Bus) FMS- Standard |  |  |  |  |  |  |  | the format ab.cd where this byte represents “a” ASCII |  |  |  |  |  |  |  | the format ab.cd where this byte represents “b” ASCII |  |  |  |  |  |  |  | the format ab.cd where this byte represents “c” ASCII |  |  |  |  |  |  |  | the format ab.cd where this byte represents “d” ASCII |  |  |  |  |  |  |  | Reserved for (Bus) FMS- Standard |  | Reserved for (Bus) FMS- Standard |  |  | Reserved for (Bus) FMS- Standard |  | values values values values |  |
|  |  |  |  |  |  |  |  |  | SPN 2806 |  |  |  |  |  |  |  | SPN 2806 |  |  |  |  |  |  |  | SPN 2806 |  |  |  |  |  |  |  | SPN 2806 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SPN |  |
|  | Requests supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name |  |
|  | 00b = request is |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | values |  |
|  | not supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | values |  |
|  | 01b= request is |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | values |  |
|  | supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | values |  |
|  | 10b = reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | values |  |
|  | 11b = don´t care |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | values |  |
|  | SPN 2805 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SPN |  |
|  | Diagnostics supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name |  |
|  | 00b = diagnostics is |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | values |  |
|  | not supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | values |  |
|  | 01b = diagnostics is |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | values |  |
|  | supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | values |  |
|  | 10b = reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | values |  |
|  | 11b = don´t care |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | values |  |
|  | SPN 2804 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SPN |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE | Scania AB Scania CV Volvo Bus Corporation Renault Trucks | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan | Name of document | FMS-Standard |  | Page | 17 (74) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) | ACEA Task Force HDEI/BCEI |  | Date 11.11.2025 | Approved final | Issue Version 05 | Reg. no. |  |
| Subject |  | FMS-Standard description |  |  |  |  |  |
| Description acc. SAE J 1939: Information which specifies the capabilities of the Fleet Management System (FMS) - standard interface device. This PGN typically is sourced from the network interconnect FMS - standard interface device. Requests supported: Status signal which indicates if the FMS Vehicle Interface (FMS Gateway) will respond to requests from the FMS device for the PGNs listed in the FMS Interface Specification. This mode is to support FMS gateway devices that only operate in a 'Request' mode. The FMS PGNs may also be broadcast periodically in this mode. The FMS Gateway will NOT support the requests for information not included in the FMS Interface Specification onto the vehicle network.. Diagnostics supported: Status signal which indicates if the FMS Vehicle Interface (FMS Gateway) supports the handling of diagnostic messages from the vehicle network onto the FMS network. The FMS gateway does NOT support the re-broadcast of diagnostics messages present on the vehicle network. If this 'FMS-standard Diagnostics Supported' feature is supported by the FMS Gateway, the FMS Gateway will support the requests for diagnostics information (from the FMS device) onto the vehicle network and pass the responses onto the FMS network. Note: This feature of the FMS Gateway is independent of the 'FMS-standard Requests Supported'. The FMS Gateway may support diagnostics without supporting the 'FMS-standard Requests Supported' function, or visa-versa.. FMS-standard SW-version supported: Information that identifies which issue level of the FMS-standard document the software included in the FMS gateway supports. Four bytes, representing ab.cd type revision level identification. Version number in the format ab.cd where byte2 and 3 represent the version number for trucks “ab” (ASCII) Byte 4 and 5 represent the version for buses and coaches “cd”(ASCII) “00” represents “not supported” For example, FMS-standard version 02.06 means the fms gateway supports version 02 of truck fms-standard and version 06 of bus fms-standard. Additional comment: See example 2.2 |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  |  |  |  |  |  |  |  |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  |  |  |  |  |  |  |  |  |  |  |  |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  |  | Page 18 (74) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  |  |  |  |  |  |  |  |  |  |  |  | ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  |  | Reg. no. |  |  |
| Subject |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1.7 High Resolution Vehicle Distance: VDHR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FEC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |  |
|  | 65,217 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |  |
|  | 1000 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |  |
|  | Data Byte 1 |  |  |  |  |  |  |  | Data Byte 2 |  |  |  |  |  |  |  | Data Byte 3 |  |  |  |  |  |  |  | Data Byte 4 |  |  |  |  |  |  |  | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  |  | Data Byte 8 |  | Byte No |  |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  |  |  | Bit No |  |
|  | High resolution total vehicle distance 5 m / Bit gain 0 m offset SPN 917 |  |  |  |  |  |  |  | High resolution total vehicle distance 5 m / Bit gain 0 m offset SPN 917 |  |  |  |  |  |  |  | High resolution total vehicle distance 5 m / Bit gain 0 m offset SPN 917 |  |  |  |  |  |  |  | High resolution total vehicle distance 5 m / Bit gain 0 m offset SPN 917 |  |  |  |  |  |  |  | Not used for (Bus) FMS-Standard |  | Not used for (Bus) FMS-Standard |  | Not used for (Bus) FMS-Standard |  |  | Not used for (Bus) FMS-Standard |  | Name values values values values SPN |  |
| Description acc. SAE J 1939: High resolution total vehicle distance: Accumulated distance travelled by the vehicle during its operation. Additional comment: The mentioned resolution is not related to the accuracy of the signal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  |  |  |  |  |  |  |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  |  |  |  |  |  |  |  |  |  |  |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Page |  |  | 19 (74) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  |  |  |  |  |  |  |  |  |  |  | ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  |  |  |  |  | Issue Version 05 |  |  |  |  |  |  |  | Reg. no. |  |  |  |  |  |
| Subject |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1.8 Tachograph : TCO1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FE6C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |  |
|  | 65,132 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |  |
|  | 20 ms/ 50 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |  |
|  | Data Byte 1 |  |  |  |  |  |  |  | Data Byte 2 |  |  |  |  |  |  |  | Data Byte 3 |  |  |  |  |  |  |  | Data Byte 4 |  |  |  |  |  |  |  | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  |  |  |  |  |  |  |  | Data Byte 8 |  |  |  |  |  |  |  | Byte No |  |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  | 8 | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Bit No. |  |
|  | Vehicle motion 00b = Vehicle motion not detected 01b = vehicle motion detected |  |  |  |  |  |  |  | Vehicle Overspeed 00b = No overspeed 01b = Overspeed |  |  |  |  |  |  |  | Not used for (Bus) FMS-Standard could be sent as “not available” or “don´t care” |  |  |  |  |  |  |  | Direction indicator 00b = Forward 01b = Reverse |  |  |  |  |  |  |  | Not used for (Bus) FMS- Standard |  | Not used for (Bus) FMS- Standard |  | Tachogr. vehicle speed 1/256 km/h Bit gain 0 km/h offset |  |  |  |  |  |  |  |  | Tachogr. vehicle speed 1/256 km/h Bit gain 0 km/h offset |  |  |  |  |  |  |  | Name values values values values |  |
|  | SPN 1611 |  |  |  |  |  |  |  | SPN 1614 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SPN 1619 |  |  |  |  |  |  |  |  |  |  |  | SPN 1624 |  |  |  |  |  |  |  |  | SPN 1624 |  |  |  |  |  |  |  | SPN |  |
|  | Driv. 2 working state 000b = Rest 001b =Driver available 010b = Work 011b = Drive 110b = Error 111b = not available |  |  |  |  |  |  |  | Driver 1 card 00b = Card not present 01b= Card present |  |  |  |  |  |  |  | Driver 2 card 00b = Card not present 01b= Card present |  |  |  |  |  |  |  | Tachograph performance 00b = Normal performance 01b = Performance analysis |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name values values values values values values values |  |
|  | SPN 1613 |  |  |  |  |  |  |  | SPN 1615 |  |  |  |  |  |  |  | SPN 1616 |  |  |  |  |  |  |  | SPN 1620 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SPN |  |
|  | Driv. 1 working state 000b = Rest 001b =Driver available 010b = Work 011b = Drive 110b = Error 111b = not available |  |  |  |  |  |  |  | Driv. 1 time rel states 0000b = normal 0001b = 15 min bef. 4 ½ h 0010b = 4 ½ h reached 0011b = 15 min bef. 9 h 0100b = 9 h reached 0101b = 15 min bef. 16 h 0110b = 16h reached 1110b = Error 1111b = not available |  |  |  |  |  |  |  | Driv 2 time rel. states 0000b = normal 0001b = 15 min bef. 4 ½ h 0010b = 4 ½ h reached 0011b=15 min before 9 h 0100b = 9 h reached 0101b = 15 min bef. 16 h 0110b = 16h reached 1110b = Error 1111b = not available |  |  |  |  |  |  |  | Handling information 00b = no handling information 01b = handling information SPN 1621 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name values values values values values values values values values |  |
|  | SPN 1612 |  |  |  |  |  |  |  | SPN 1617 |  |  |  |  |  |  |  | SPN 1618 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SPN |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | System event 00b = no tachogr. Event 01b = tachogr. Event |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name values values |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SPN 1622 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SPN |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE | Scania AB Scania CV Volvo Bus Corporation Renault Trucks | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan | Name of document | FMS-Standard |  | Page | 20 (74) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) | ACEA Task Force HDEI/BCEI |  | Date 11.11.2025 | Approved final | Issue Version 05 | Reg. no. |  |
| Subject |  | FMS-Standard description |  |  |  |  |  |
| Description acc. SAE J 1939: Vehicle motion: Indicates whether motion of the vehicle is detected or not. Driver 2 Working State: State of work of the driver. Driver 1 Working State: State of work of the driver. Vehicle Overspeed: Indicates whether the vehicle is exceeding the legal speed limit set in the tachograph. Driver 1 Card: Indicates the presence of a driver card. Driver 1 Time Related Status: Indicates if the driver approaches or exceeds working time limits (or other limits). Driver 2 Card: Indicates the presence of a driver card. Driver 2 Time Related Status: Indicates if the driver approaches or exceeds working time limits (or other limits). Direction Indicator: Indicates the direction of the vehicle. Tachograph Performance: Indicates the tachograph performance; including electronic or mechanical analysis, instrument analysis, speed sensor analysis, mass storage analysis, and printer analysis. Handling Information: Indicates that handling information is present. Information could include 'no printer paper', 'no driver card', etc. System Event: Indicates that a tachograph event has occurred. This may include power supply interruption, interruption of the speed sensor, incorrect data on the driver card, driving without a driver card, illegal removal of a driver card, insertion of a driver card during driving, and time adjustment. Tachograph Vehicle Speed: Speed of the vehicle registered by the tachograph. Additional comment: Tachograph vehicle speed might differ from the wheel based speed The availability of the value direction indicator (SPN 1619) is tachograph dependant. At the issuing date of this document the tachographs are not supporting this value. Only available if a digital tachograph is present |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  |  |  |  |  |  |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  | Name of document FMS-Standard |  |  |  |  |  |  | Page 21 (74) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  |  |  |  |  |  |  |  | ACEA Task Force HDEI/BCEI |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  |  | Reg. no. |  |  |
| Subject |  |  |  |  |  |  |  |  |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |
| 1.1.9 Engine Temperature 1: ET1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FEEE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |  |
|  | 65,262 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |  |
|  | 1000 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |  |
|  | Data Byte 1 |  |  |  |  |  |  |  | Data Byte 2 |  | Data Byte 3 | Data Byte 4 |  | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  |  | Data Byte 8 |  | Byte No |  |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Bit No |  |
|  | Engine coolant temperature 1 °C / Bit gain - 40 °C offset SPN 110 |  |  |  |  |  |  |  | Not used for (Bus) FMS-Standard |  | Not used for FMS-Standard | Not used for FMS-Standard |  | Not used for (Bus) FMS-Standard |  | Not used for (Bus) FMS-Standard |  | Not used for (Bus) FMS-Standard |  |  | Not used for (Bus) FMS-Standard |  | Name values values values SPN |  |
| Description acc. SAE J 1939: Engine Coolant Temperature: Temperature of liquid found in engine cooling system. Additional comment: The mentioned resolution is not related to the accuracy of the signal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  |  | Page 22 (74) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  |  | Approved final |  | Issue Version 05 |  | Reg. no. |  |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1.10 Ambient Conditions: AMB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FEF5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |  |
|  | 65,269 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |  |
|  | 1000 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |  |
|  | Data Byte 1 | Data Byte 2 | Data Byte 3 | Data Byte 4 |  |  |  |  |  |  |  | Data Byte 5 |  |  |  |  |  |  |  | Data Byte 6 |  | Data Byte 7 |  | Data Byte 8 |  | Byte No |  |
|  |  |  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  | Bit No |  |
|  | Not used for (Bus) FMS-Standard | Not used for FMS-Standard | Not used for FMS-Standard | Ambient Air Temperature 0.03125 °C / Bit gain - 273 °C offset SPN 171 |  |  |  |  |  |  |  | Ambient Air Temperature 0.03125 °C / Bit gain - 273 °C offset SPN 171 |  |  |  |  |  |  |  | Not used for (Bus) FMS-Standard |  | Not used for (Bus) FMS-Standard |  | Not used for (Bus) FMS-Standard |  | Name Name values values values values SPN |  |
| Description acc. SAE J 1939: Ambient Air Temperature: Temperature of air surrounding vehicle. Additional comment: The mentioned resolution is not related to the accuracy of the signal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  | Page 23 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  | Reg. no. |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1.11 Driver's Identification: DI |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FE6B |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 65,131 |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 10000 ms |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Variable 1-n | Variable 1-n | Variable 1-n | Variable 1-n | Variable 1-n |  | Variable 1-n |  | Variable 1-n |  | Variable 1-n |  | Byte No |
|  | 8-1 | 8 - 1 | 8 – 1 | 8-1 | 8 - 1 |  | 8 - 1 |  | 8 - 1 |  | 8 - 1 |  | Bit No. |
|  | Driver 1 identification Driver 2 identification SPN 1625/1626 | Driver 1 identification Driver 2 identification SPN 1625/1626 | Driver 1 identification Driver 2 identification SPN 1625/1626 | Driver 1 identification Driver 2 identification SPN 1625/1626 | Driver 1 identification Driver 2 identification SPN 1625/1626 |  | Driver 1 identification Driver 2 identification SPN 1625/1626 |  | Driver 1 identification Driver 2 identification SPN 1625/1626 |  | Driver 1 identification Driver 2 identification SPN 1625/1626 |  | Name Name values values values values SPN |
| Description acc. SAE J 1939: Field: a Driver 1 Identification Delimiter (ASCII '*') b Driver 2 Identification Delimiter (ASCII '*') NOTE - If only driver card 1 is present, only the parameter driver 1 identification and two delimiters shall be transmitted. If only driver card 2 is present, a delimiter followed by parameter driver 2 identification and the second delimiter shall be transmitted. If no driver cards are present, only the two delimiters shall be sent." Additional comment: The driver ID is only available if a digital tachograph is present. Driver ID = Issuing member state + CardNumber = 3 + 16 Byte (acc. ISO 16844) If a driver ID is available the message is sent with a Broadcast Announce Message (BAM) If no driver cards are present then it is broadcasted with PGN 00FE6B (8Byte) containing two delimiters and filled with “FF” at the unused bytes. Difference to SAE: broadcast instead of on request The information is sent by the tachograph. Depending on the tachograph brand the information is not immediately available after insertion of the driver card. |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  | Page 24 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  | Reg. no. |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1.12 Fuel Economy: LFE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FEF2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 65,266 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 100 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 |  |  |  |  |  |  |  | Data Byte 2 |  |  |  |  |  |  |  | Data Byte 3 |  |  |  |  |  |  |  | Data Byte 4 |  |  |  |  |  |  |  | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  | Data Byte 8 |  | Byte No |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  |  | Bit No. |
|  | Fuel Rate 0.05 L/h per bit 0 offset 0 to 3,212.75 L/h SPN 183 |  |  |  |  |  |  |  | Fuel Rate 0.05 L/h per bit 0 offset 0 to 3,212.75 L/h SPN 183 |  |  |  |  |  |  |  | Instantaneous Fuel Economy 1/512 km/L per bit 0 offset 0 to 125,5 km/L SPN 184 |  |  |  |  |  |  |  | Instantaneous Fuel Economy 1/512 km/L per bit 0 offset 0 to 125,5 km/L SPN 184 |  |  |  |  |  |  |  | Not used for (Bus) FMS-Standard |  | Not used for (Bus) FMS-Standard |  | Not used for (Bus) FMS-Standard |  | Not used for (Bus) FMS-Standard |  | Name Name values values values values values SPN |
| Description acc. SAE J 1939: Fuel rate: Amount of fuel consumed by engine per unit of time Instantaneous Fuel Economy: Current fuel economy at current vehicle velocity Additional comment: Calculated values given as indications, not as contractual values. The mentioned resolution is not related to the accuracy of the signal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  | Page 25 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  | Reg. no. |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1.13 Air Supply Pressure : AIR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FEAE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 65,198 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 1000 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 | Data Byte 2 | Data Byte 3 |  |  |  |  |  |  |  | Data Byte 4 |  |  |  |  |  |  |  | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  | Data Byte 8 |  | Byte No |
|  |  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  |  | Bit No. |
|  | Not used for (Bus) FMS-Standard | Not used for (Bus) FMS-Standard | Service Brake Air Pressure Circuit #1 8 kPa/Bit 0 offset SPN 1087 |  |  |  |  |  |  |  | Service Brake Air Pressure Circuit #2 8 kPa/Bit 0 offset SPN 1088 |  |  |  |  |  |  |  | Not used for (Bus) FMS- Standard |  | Not used for (Bus) FMS- Standard |  | Not used for (Bus) FMS- Standard |  | Not used for (Bus) FMS- Standard |  | Name Name values values values values SPN |
| Description acc. SAE J 1939: Service Brake Air Pressure Circuit #1: The pneumatic pressure in the service brake circuit or reservoir #1. Service Brake Air Pressure Circuit #2: The pneumatic pressure in the service brake circuit or reservoir #2. Additional comment: The mentioned resolution is not related to the accuracy of the signal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  | Page 26 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  | Reg. no. |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1.14 High Resolution Fuel Consumption (Liquid): HRLFC |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FD09 |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 64,777 |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 1000 ms |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 | Data Byte 2 | Data Byte 3 | Data Byte 4 | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  | Data Byte 8 |  | Byte No |
|  |  |  |  |  | Bit 8 - 1 |  | Bit 8 - 1 |  | Bit 8 - 1 |  | Bit 8 - 1 |  | Bit No. |
|  | Not used for (Bus) FMS-Standard | Not used for (Bus) FMS-Standard | Not used for (Bus) FMS-Standard | Not used for (Bus) FMS-Standard | High resolution engine total fuel used 0.001 L/bit 0 offset 0 to 4,211,081.215 L SPN 5054 |  | High resolution engine total fuel used 0.001 L/bit 0 offset 0 to 4,211,081.215 L SPN 5054 |  | High resolution engine total fuel used 0.001 L/bit 0 offset 0 to 4,211,081.215 L SPN 5054 |  | High resolution engine total fuel used 0.001 L/bit 0 offset 0 to 4,211,081.215 L SPN 5054 |  | Name Name values values values values values SPN |
| Description acc. SAE J 1939: Engine fuel consumption accumulators High resolution engine total fuel used: Accumulated amount of fuel used during vehicle operation. High resolution used for calculations and fleet management systems. Additional comment: Is implemented if technical possible. Calculated values given as indications, not as contractual values. The mentioned resolution is not related to the accuracy of the signal. |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  | Page 27 (74) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  | Reg. no. |  |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1.15 Aftertreatment 1 Diesel Exhaust Fluid Tank 1 Information: AT1T1I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FE56 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |  |
|  | 65,110 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |  |
|  | 1000 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |  |
|  | Data Byte 1 |  |  |  |  |  |  |  | Data Byte 2 | Data Byte 3 | Data Byte 4 | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  | Data Byte 8 |  | Byte No |  |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  |  |  |  |  | Bit No |  |
|  | Aftertreatment 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name |  |
|  | Diesel Exhaust Fluid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Tank 1 Level 0 % = Empty |  |  |  |  |  |  |  | Not used for | Not used for | Not used for | Not used for |  | Not used for |  | Not used for |  | Not used for |  | values values |  |
|  | 100% = Full |  |  |  |  |  |  |  | (Bus) FMS-Standard | (Bus) FMS-Standard | (Bus) FMS-Standard | (Bus) FMS-Standard |  | (Bus) FMS-Standard |  | (Bus) FMS-Standard |  | (Bus) FMS-Standard |  | values |  |
|  | 0.4 % / bit, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | values |  |
|  | 0 offset |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | values |  |
|  | SPN 1761 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SPN |  |
| Description acc. SAE J 1939: Ratio of volume of diesel exhaust fluid to the total volume of diesel exhaust fluid storage container. Additional comment: The mentioned resolution is not related to the accuracy of the signal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan | Name of document FMS-Standard |  |  | Page 28 (74) |
| --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI | Date 11.11.2025 | Approved final | Issue Version 05 | Reg. no. |
| Subject FMS-Standard description |  |  |  |  |
| 1.1.16 FMS Tell Tale Status: FMS1 |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  |  |  | Name of document FMS-Standard |  |  |  |  |  | Page 29 (74) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  |  |  | ACEA Task Force HDEI/BCEI |  |  |  |  |  | Date 11.11.2025 |  |  | Approved final |  | Issue Version 05 | Reg. no. |  |  |
| Subject |  |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |
| Table for Telltale status: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Block ID | Telltale Status | Telltale ID | ISO No. | Name | Mandatory Truck only |  | Block ID | Telltale Status |  | Telltale ID | ISO No. |  | Name |  |  | Mandatory Truck only |  |
|  | 0 | 1 | 1 | 27 | Cooling air conditioning |  |  | 2 | 1 |  | 31 | 2441 |  | Steering failure |  |  |  |  |
|  | 0 | 2 | 2 | 82 | High beam, main beam |  |  | 2 | 2 |  | 32 | 2461 |  | Height Control (Levelling) |  |  |  |  |
|  | 0 | 3 | 3 | 83 | Low beam, dipped beam |  |  | 2 | 3 |  | 33 | 2574 |  | Retarder |  |  |  |  |
|  | 0 | 4 | 4 | 84 | Turn signals |  |  | 2 | 4 |  | 34 | 2596 |  | Engine Emission system failure (Mil indicator) |  |  |  |  |
|  | 0 | 5 | 5 | 85 | Hazard warning |  |  | 2 | 5 |  | 35 | 2630 |  | ESC indication |  |  |  |  |
|  | 0 | 6 | 6 | 100 | Provision for the disabled or handicapped persons |  |  | 2 | 6 |  | 36 | no |  | Brake lights |  |  |  |  |
|  | 0 | 7 | 7 | 238 | Parking Brake | X |  | 2 | 7 |  | 37 | no |  | Articulation |  |  |  |  |
|  | 0 | 8 | 8 | 239 | Brake failure/brake system malfunction |  |  | 2 | 8 |  | 38 | no |  | Stop Request |  |  |  |  |
|  | 0 | 9 | 9 | 242 | Hatch open |  |  | 2 | 9 |  | 39 | no |  | Pram request |  |  |  |  |
|  | 0 | 10 | 10 | 245 | Fuel level | X |  | 2 | 10 |  | 40 | no |  | Bus stop brake |  |  |  |  |
|  | 0 | 11 | 11 | 246 | Engine coolant temperature | X |  | 2 | 11 |  | 41 | 2946 |  | AdBlue level |  |  |  |  |
|  | 0 | 12 | 12 | 247 | Battery charging condition |  |  | 2 | 12 |  | 42 | no |  | Raising |  |  |  |  |
|  | 0 | 13 | 13 | 248 | Engine oil | X |  | 2 | 13 |  | 43 | no |  | Lowering |  |  |  |  |
|  | 0 | 14 | 14 | 456 | Position lights,side lights |  |  | 2 | 14 |  | 44 | no |  | Kneeling |  |  |  |  |
|  | 0 | 15 | 15 | 633 | Front fog light |  |  | 2 | 15 |  | 45 | no |  | Engine compartment temperature |  |  |  |  |
|  | 1 | 1 | 16 | 634 | Rear fog light |  |  | 3 | 1 |  | 46 | no |  | Auxillary air pressure |  |  |  |  |
|  | 1 | 2 | 17 | 637 | Park Heating |  |  | 3 | 2 |  | 47 | 2432 |  | Air filter clogged |  |  |  |  |
|  | 1 | 3 | 18 | 640 | Engine / Mil indicator | X |  | 3 | 3 |  | 48 | 2452 |  | Fuel filter differential pressure |  |  |  |  |
|  | 1 | 4 | 19 | 717 | Service, call for maintenance |  |  | 3 | 4 |  | 49 | 249 |  | Seat belt |  |  |  |  |
|  | 1 | 5 | 20 | 1168 | Transmission fluid temperature |  |  | 3 | 5 |  | 50 | no |  | EBS |  |  |  |  |
|  | 1 | 6 | 21 | 1396 | Transmission failure/malfunction |  |  | 3 | 6 |  | 51 | 2682 |  | Lane departure indication |  |  |  |  |
|  | 1 | 7 | 22 | 1407 | Anti-lock brake system failure |  |  | 3 | 7 |  | 52 | no |  | Advanced emergency braking system |  |  |  |  |
|  | 1 | 8 | 23 | 1408 | Worn brake linings |  |  | 3 | 8 |  | 53 | 2581 |  | ACC |  |  |  |  |
|  | 1 | 9 | 24 | 1422 | Windscreen washer fluid/windshield washer fluid |  |  | 3 | 9 |  | 54 | no |  | Trailer connected |  |  |  |  |
|  | 1 | 10 | 25 | 1434 | Tire failure/malfunction |  |  | 3 | 10 |  | 55 | 2444/2445 |  | ABS Trailer 1,2 |  |  |  |  |
|  | 1 | 11 | 26 | 1603 | Malfunction/general failure |  |  | 3 | 11 |  | 56 | 2108 |  | Airbag |  |  |  |  |
|  | 1 | 12 | 27 | 2426 | Engine oil temperature |  |  | 3 | 12 |  | 57 | no |  | EBS Trailer 1,2 |  |  |  |  |
|  | 1 | 13 | 28 | 2427 | Engine oil level |  |  | 3 | 13 |  | 58 | no |  | Tachograph indication |  |  |  |  |
|  | 1 | 14 | 29 | 2429 | Engine coolant level |  |  | 3 | 14 |  | 59 | 2649 |  | ESC switched off |  |  |  |  |
|  | 1 | 15 | 30 | 2440 | Steering fluid level |  |  | 3 | 15 |  | 60 | no |  | Lane departure warning switched off |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  |  |  | Name of document FMS-Standard |  |  |  |  |  | Page 30 (74) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  |  |  | ACEA Task Force HDEI/BCEI |  |  |  |  |  | Date 11.11.2025 |  |  | Approved final |  | Issue Version 05 | Reg. no. |  |  |
| Subject |  |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Block ID | Telltale Status | Telltale ID | ISO No. | Name | Mandatory Truck only |  | Block ID | Telltale Status |  | Telltale ID | ISO No. |  | Name |  |  | Mandatory Truck only |  |
|  | 4 | 1 | 61 | 2433 | Engine emission filter (Soot Filter) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | 2 | 62 | 2633 | Electric motor failures |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | 3 | 63 | no | AdBlue tampering |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | 4 | 64 | no | Multiplex System |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | 5 | 65 | 2632 | Battery pack |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | 6 | 66 | 6042 | High voltage system caution |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | 7 | 67 | 3129 | Battery pack temperature |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | 8 | 68 | 2639 | Limited performance electric motor |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | 9 | 69 | 2455 | Battery pack cooling |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | 10 | 70 | no | Charging status |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | 11 | 71 | no | Traction battery balancing |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | 12 | 72 | no | Pantograph status |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | 13 | 73 | no | Fire detection / alarm |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | 14 | 74 | no | Recuperation disturbed |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | 15 | 75 | no | High Voltage Battery Thermal Event |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE | Scania AB Scania CV Volvo Bus Corporation Renault Trucks | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan | Name of document | FMS-Standard |  | Page | 31 (74) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) | ACEA Task Force HDEI/BCEI |  | Date 11.11.2025 | Approved final | Issue Version 05 | Reg. no. |  |
| Subject |  | FMS-Standard description |  |  |  |  |  |
| Annotations: Tell Tale Status: The Tell Tale Status information is derived from information displayed to the driver’s dashboard. The tell tale number is related to the symbols in the ISO 7000 document and not to the description. The not related to ISO 7000 is stated with “no” There are three possible conditions stated: Red (“R”), Yellow (“Y”), Info (“I”). The interpretation of the status is manufacturer dependant and might be different. For details please refer to the manufacturers’ document. The symbols used in the dash display of each manufacturer might vary from ISO symbols. The lamp characteristic (e.g., flashing) is not reflected in the tell tale information. Some information might be not shown to the driver as a tell-tale. The status information is present as long the status is valid. Additional comment: Due to the repetition rate of the message, it is not guaranteed to include all intermittent signals.* The message is mandatory for block 0-1 (for trucks) however some tell tale status information are not mandatory Further blocks are optional “skipping blocks” (e.g., if all tell tales of a block are not available) is allowed (e.g. sending block 0,1,4) Charging status: The information is from the vehicle point of view and might be different from the SPN 13171 which is the view from the charging station Traction battery balancing: The balancing is mainly done automatically or initiated from the battery system and might be needed to be connected to a charging station. Depending on the OEM of the vehicle the balancing might be always on (as well during driving) and handled by the vehicle system. Pantograph status: indication if the pantograph has a fault/warning Fire detection / alarm: Extinguisher is not included, Yellow state could be used for indicating that there is a problem with the fire detection system Recuperation disturbed: Indicate that recuperation is disabled/disturbed/limited in BEVs. High Voltage Battery Thermal Event: Information about a possible thermal issue detection. see example in 2.3. |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  | Page 32 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  | Reg. no. |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1.17 Electronic Brake Controller 1: EBC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00F001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 61,441 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 100 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 | Data Byte 2 |  |  |  |  |  |  |  | Data Byte 3 | Data Byte 4 | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  | Data Byte 8 |  | Byte No |
|  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  |  |  |  | Bit No. |
|  | Not used for (Bus) FMS-Standard | Brake Pedal Position 0,4 % / Bit gain 0 % offset SPN 521 |  |  |  |  |  |  |  | Not used for (Bus) FMS-Standard | Not used for (Bus) FMS-Standard | Not used for (Bus) FMS- Standard |  | Not used for (Bus) FMS- Standard |  | Not used for (Bus) FMS- Standard |  | Not used for (Bus) FMS- Standard |  | Name Name values values values values SPN |
| Description acc. SAE J 1939: Ratio of brake pedal position to maximum pedal position. Used for electric brake applications. 0% means no braking. Also when there are two brake pedals on the machine (Left Brake Pedal Position SPN 3033 and Right Brake Pedal Position SPN 3032) the maximum of the two should be transmitted for Brake Pedal Position. Additional comment: The mentioned resolution is not related to the accuracy of the signal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  |  |  |  |  |  |  | Page 33 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  |  |  |  | Issue Version 05 |  |  |  |  | Reg. no. |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1.18 Electronic Engine Controller 14: EEC14 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FDC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 64,962 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 10000 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 | Data Byte 2 | Data Byte 3 | Data Byte 4 | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  |  |  |  |  |  |  | Data Byte 8 |  | Byte No |
|  |  |  |  |  |  |  |  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  | Bit No. |
|  | Not used for (Bus) FMS-Standard | Not used for (Bus) FMS-Standard | Not used for (Bus) FMS-Standard | Not used for (Bus) FMS-Standard | Not used for (Bus) FMS- Standard |  | Not used for (Bus) FMS- Standard |  | Fuel Type 256 states/8 bit SPN 5837 |  |  |  |  |  |  |  | Not used for (Bus) FMS- Standard |  | Name Name values values SPN |
| Description acc. SAE J 1939: Type of fuel currently being utilized by the vehicle. See table below. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  | Nam | e of document |  | FMS-Standard |  | Page 34 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  | ACEA Task Force HDEI/BCEI |  |  | Date 11.11.2025 |  |  | Approved final | Issue Version 05 | Reg. no. |  |
| Subject |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |
| Data Byte | Scaling |  | External Display | Data Byte |  |  | Scaling |  |  |  | External Display |
| 0x00h | Not available |  | NONE | 0x12h |  |  | Hybrid vehicle using gasoline engine on ethanol |  |  |  | HYB_ETH |
| 0x01h | Gasoline/petrol |  | GAS | 0x13h |  |  | Hybrid vehicle using diesel engine |  |  |  | HYB_DSL |
| 0x02h | Methanol |  | METH | 0x14h |  |  | Hybrid vehicle using battery |  |  |  | HYB_ELEC |
| 0x03h | Ethanol |  | ETH | 0x15h |  |  | Hybrid vehicle using battery and combustion engine |  |  |  | HYB_MIX |
| 0x04h | Diesel |  | DSL | 0x16h |  |  | Hybrid vehicle in regeneration mode |  |  |  | HYB_REG |
| 0x05h | Liquefied Petroleum Gas (LPG) |  | LPG | 0x17h |  |  | Natural Gas (NG) |  |  |  | NG |
| 0x06h | Compressed Natural Gas (CNG) |  | CNG | 0x18h |  |  | Bi-fuel vehicle using NG |  |  |  | BI_NG |
| 0x07h | Propane |  | PROP | 0x19h |  |  | Bi-fuel using diesel |  |  |  |  |
| 0x08h | Battery/electric |  | ELEC | 0x1Ah |  |  | Natural Gas (Compressed or Liquefied Natural Gas) |  |  |  |  |
| 0x09h | Bi-fuel vehicle using gasoline |  | BI_GAS | 0x1Bh |  |  | Dual Fuel – Diesel and CNG |  |  |  |  |
| 0x0Ah | Bi-fuel vehicle using methanol |  | BI_METH | 0x1Ch |  |  | Dual Fuel – Diesel and LNG |  |  |  |  |
| 0x0Bh | Bi-fuel vehicle using ethanol |  | BI_ETH | 0x1Dh |  |  | Fuel Cell Utilizing Hydrogen |  |  |  |  |
| 0x0Ch | Bi-fuel vehicle using LPG |  | BI_LPG | 0x1Eh |  |  | Hydrogen Internal Combustion Engine |  |  |  |  |
| 0x0Dh | Bi-fuel vehicle using CNG |  | BI_CNG | 0x1Fh |  |  | Kerosene |  |  |  |  |
| 0x0Eh | Bi-fuel vehicle using propane |  | BI_PROP | 0x20h |  |  | Heavy Fuel Oil |  |  |  |  |
| 0x0Fh | Bi-fuel vehicle using battery |  | BI_ELEC | 0x21h- 0xFDh |  |  | ISO/SAE reserved |  |  |  |  |
| 0x10h | Bi-fuel vehicle using battery and combustion engine |  | BI_MIX | 0xFEh |  |  | Error |  |  |  |  |
| 0x11h | Hybrid vehicle using gasoline engine |  | HYB_GAS | 0xFFh |  |  | Not available (Parameter not supported) |  |  |  |  |
| EXAMPLE: If a Bi-fuel vehicle has less than 10% ethanol in the gasoline/petrol, then the external test equipment shall display state 0x09. In such a case, the system would be using gasoline/petrol (fuel) tables and OBD thresholds. SPN 5458 should either artificially display 0% or some number 10% or less if the system has such resolution. If the ethanol in the gasoline/petrol is above 10%, then the external test equipment shall display state 0x0B. SPN 5458 shall report the calculated ethanol/alcohol percentage. Note: "Not available" (Data 0x00) shall be used for the following conditions where no fuel is being utilized: - Ignition Key On/Engine Off, for example, for bi-fuel vehicles when the vehicle cannot identify which fuel is being used before engine start - Engine stopped for vehicle using Start/Stop strategy or for hybrid vehicle with engine stopped If FUEL_TYP is utilized on a single fuel vehicle, it can continue to indicate the previously utilized fuel type while the engine is in Decel Fuel Cut Off (DFCO) mode. It is not required to indicate 0x00 while in DFCO. |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE | Scania AB Scania CV Volvo Bus Corporation Renault Trucks | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan | Name of document | FMS-Standard |  | Page | 35 (74) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) | ACEA Task Force HDEI/BCEI |  | Date 11.11.2025 | Approved final | Issue Version 05 | Reg. no. |  |
| Subject |  | FMS-Standard description |  |  |  |  |  |
| Regarding fuel type 0x17 - Natural Gas - this applies to any type of natural gaseous fuel when the source (CNG, LPG, or other) is not relevant. Regarding fuel type 0x18 - Bi-fuel vehicle using NG; this applies to bi-fuel vehicles running Diesel and Natural Gas, where the source of the natural gaseous fuel is not relevant. Regarding fuel type 20h - Heavy Fuel Oil is a Maritime fuel classification for pure or nearly pure residual oil, roughly equivalent to No. 6 fuel oil Marine diesel oil contains some heavy fuel oil, unlike regular diesels. |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  | Page 36 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  | Reg. no. |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1.19 Fuel Consumption (Gaseous): GFC |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FEAF |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 65,199 |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 1000 ms |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 | Data Byte 2 | Data Byte 3 | Data Byte 4 | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  | Data Byte 8 |  | Byte No |
|  |  |  |  |  | Bit 8-1 |  | Bit 8-1 |  | Bit 8-1 |  | Bit 8-1 |  | Bit No. |
|  | Not used for (Bus) FMS-Standard | Not used for (Bus) FMS-Standard | Not used for (Bus) FMS-Standard | Not used for (Bus) FMS-Standard | Total Fuel Used (Gaseous) 0.5 kg/bit 0 to 2,105,540,607.5 kg SPN 1040 |  | Total Fuel Used (Gaseous) 0.5 kg/bit 0 to 2,105,540,607.5 kg SPN 1040 |  | Total Fuel Used (Gaseous) 0.5 kg/bit 0 to 2,105,540,607.5 kg SPN 1040 |  | Total Fuel Used (Gaseous) 0.5 kg/bit 0 to 2,105,540,607.5 kg SPN 1040 |  | Name Name values values values values SPN |
| Description acc. SAE J 1939: Total fuel consumed (trip drive fuel + trip PTO governor moving fuel + trip PTO governor non-moving fuel + trip idle fuel) over the life of the engine. Additional comment: The mentioned resolution is not related to the accuracy of the signal |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  |  |  |  |  |  |  |  | Page 37 (74) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  |  |  |  |  | Issue Version 05 |  |  |  |  | Reg. no. |  |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1.20 Electronic Retarder Controller 1: ERC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00F000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |  |
|  | 61,440 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |  |
|  | 100 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |  |
|  | Data Byte 1 |  |  |  |  |  |  |  | Data Byte 2 |  |  |  |  |  |  |  | Data Byte 3 | Data Byte 4 | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  |  |  |  |  |  |  |  | Data Byte 8 |  | Byte No |  |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  | 8 | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 |  |  | Bit No |  |
|  | Retarder Torque |  |  |  |  |  |  |  | Actual Retarder - |  |  |  |  |  |  |  |  |  |  |  |  |  | Retarder Selection, |  |  |  |  |  |  |  |  |  |  | Name |  |
|  | Mode |  |  |  |  |  |  |  | Percent Torque |  |  |  |  |  |  |  | Not used | Not used | Not used |  | Not used |  | non-engine |  |  |  |  |  |  |  |  | Not used |  | values |  |
|  | 16 states/4 bit, |  |  |  |  |  |  |  | 1 %/bit, |  |  |  |  |  |  |  | for FMS-Standard | for FMS-Standard | for FMS-Standard |  | for FMS-Standard |  | 0.4 %/bit, |  |  |  |  |  |  |  |  | for FMS-Standard |  | values |  |
|  | 0 offset |  |  |  |  |  |  |  | -125 % offset |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 % offset |  |  |  |  |  |  |  |  |  |  | values |  |
|  | SPN 900 |  |  |  |  |  |  |  | SPN 520 |  |  |  |  |  |  |  |  |  |  |  |  |  | SPN 1716 |  |  |  |  |  |  |  |  |  |  | SPN |  |
|  | Not used |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | for FMS-Standard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description acc. SAE J 1939: Retarder Torque Mode: State signal which indicates which retarder torque mode is currently generating, limiting, or controlling the torque. Note that the modes are not in prioritized order. Not all modes may be relevant for a given device. Some devices may not implement all functions. Mode 0000b means “No request”: retarder torque = 0 (no braking). See Appendix D in SAE documentation Modes 0001b to 1110b indicate that there is either a torque request or the identified function is currently controlling the retarder: retarder torque may range from 0 (no braking) to the upper limit. Actual Retarder - Percent Torque: Actual braking torque of the retarder as a percent of retarder configuration reference torque SPN 556. Retarder Selection, non-engine: The “Retarder Selection, non-engine” is the position of the driver’s selector for retarders that are not part of the engine system, expressed as percent and determined by the ratio of current position to the maximum possible position. The physical device may be a lever, rotary dial, combination of switches, or other device that the driver can use to select the type or amount of retardation needed. Additional comment: For SPN 1716: The value is related to the driver’s selection of a retarder (engine and/or drive line). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  | Page 38 (74) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  | Reg. no. |  |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1.21 Vehicle Weight: VW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FEEA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |  |
|  | 65,258 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |  |
|  | 1000 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |  |
|  | Data Byte 1 |  |  |  |  |  |  |  | Data Byte 2 |  |  |  |  |  |  |  | Data Byte 3 |  |  |  |  |  |  |  | Data Byte 4 | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  | Data Byte 8 |  | Byte No |  |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  |  |  | Bit No |  |
|  | Axle location Bit-mapped position number counting front to back facing forward Fh = not available |  |  |  |  |  |  |  | Axle weight 0.5 kg / Bit gain 0 kg offset |  |  |  |  |  |  |  | Axle weight 0.5 kg / Bit gain 0 kg offset |  |  |  |  |  |  |  | Not used for FMS-Standard | Not used for FMS-Standard |  | Not used for FMS-Standard |  | Not used for FMS-Standard |  | Not used for FMS-Standard |  | Name values values values values values values |  |
|  | SPN 928 |  |  |  |  |  |  |  | SPN 582 |  |  |  |  |  |  |  | SPN 582 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SPN |  |
|  | Tire location Bit-mapped counting left to right facing forward Fh = not available |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name values values values values values values |  |
|  | SPN 928 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SPN |  |
| Description acc. SAE J 1939: Axle / Tire Location: To identify to which of several similar devices (such as tires or fuel tanks) the information applies. The low order 4 bits represent a position number, counting left to right when facing in the direction of normal vehicle travel (forward). The high order 4 bits represent a position number, counting front to back on the vehicle. The value 0xFF indicates not available. It is recommended that output devices add 1 to the position number (range 1 to 15, not 0 to 14) for use by drivers and service technicians. Examples: Tire pressure for location 0x00 would be left front tire. Tire pressure for location 0x23 would be right outside rear rear on a 3-axle tractor with dual axle per side (3rd axle, 4th tire) Axel weight: Total mass imposed by the tires on the road surface at the specified axle. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  | Page 39 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  | Reg. no. |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1.22 EVSE 1 DC Status 1 : EVSE1DCS1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FAB8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 64,184 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 1000 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 |  |  |  |  |  |  |  | Data Byte 2 | Data Byte 3 | Data Byte 4 | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  | Data Byte 8 |  | Byte No |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  |  |  |  |  | Bit No. |
|  | DC Charging State 0 = Idle. No EVSE connected. 1 = Charging. 2 = Standby. 3-9 = reserved 10 = Initialising 11 = Pre-charge 12 = Terminating 13 = Battery failure. An error condition due to the battery state (high temperature, etc.) 14 = EVSE fault. An error condition due to the EVSE. 15 = Not available. SPN 13171 |  |  |  |  |  |  |  | Not used in FMS-Standard | Not used in FMS-Standard | Not used in FMS-Standard | Not used in FMS-Standard |  | Not used in FMS-Standard |  | Not used in FMS-Standard |  | Not used in FMS-Standard |  | Name values values values values values values values values values values values values values SPN |
| Description acc. SAE J 1939: DC Charging State: Overall State of the DC charging system. Additional Comment: Please note that the information is reported by the EVSE (Electric Vehicle Supply Equipment) beside status 0 and 15. The mentioned resolution is not related to the accuracy of the signal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  | Page 40 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  | Reg. no. |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1.23 EV Operator Indicators 1 : EVOI1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FC69 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 64,617 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | Every 10 s and on change but no faster than 1 s |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 |  |  |  |  |  |  |  | Data Byte 2 |  |  |  |  |  |  |  | Data Byte 3 | Data Byte 4 | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  | Data Byte 8 |  | Byte No |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  |  |  |  | Bit No. |
|  | HVESS Estimated Remaining Distance 0.25 km/bit 0 offset SPN 15268 |  |  |  |  |  |  |  | HVESS Estimated Remaining Distance 0.25 km/bit 0 offset SPN 15268 |  |  |  |  |  |  |  | Not used in FMS-Standard | Not used in FMS-Standard | Not used in FMS-Standard |  | Not used in FMS-Standard |  | Not used in FMS-Standard |  | Not used in FMS-Standard |  | Name Name values values SPN |
| Description acc. SAE J 1939: HVESS Estimated Remaining Distance: Estimated remaining distance based on existing available HVES system capacity. Additional Comment: The mentioned resolution is not related to the accuracy of the signal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  | Page 41 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  | Reg. no. |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1.24 Tire Condition Message 1 : TIRE1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FEF4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 65,268 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 10.000 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 |  |  |  |  |  |  |  | Data Byte 2 |  |  |  |  |  |  |  | Data Byte 3 | Data Byte 4 | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  | Data Byte 8 |  | Byte No |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  |  |  |  | Bit No. |
|  | Tire Location 256 states/8 bit 0 offset SPN 929 |  |  |  |  |  |  |  | Tire Pressure 4 kPa/bit 0 offset SPN 241 |  |  |  |  |  |  |  | Not used in FMS-Standard | Not used in FMS-Standard | Not used in FMS-Standard |  | Not used in FMS-Standard |  | Not used in FMS-Standard |  | Not used in FMS-Standard |  | Name values values SPN |
| Description acc. SAE J 1939: Tire Location: Identifies which tire is associated with the parametric data in this PG. The low order 4 bits represent a position number, counting left to right when facing in the direction of normal vehicle travel (forward). The high order 4 bits represent a position number, counting front to back on the vehicle. The value FFh indicates not available. It is recommended that output devices add 1 to the position number (range 1 to 15, not 0 to 14) for use by drivers and service technicians. Examples: Tire pressure for location 00h would be left front tire. Tire pressure for location 23h would be right outside rear rear tire on a 3-axle tractor with dual axle per side (3rd axle, 4th tire). Tire Pressure: Pressure at which air is contained in cavity formed by tire and rim. Additional Comment: The mentioned resolution is not related to the accuracy of the signal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  | Page 42 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  | Reg. no. |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1.25 Operator Indicators : OI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FEFF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 65,279 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 10 s or on change but no more often than 1 s |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 |  |  |  |  |  |  |  | Data Byte 2 | Data Byte 3 |  |  |  |  |  |  |  | Data Byte 4 |  |  |  |  |  |  |  | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  | Data Byte 8 |  | Byte No |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  |  | Bit No. |
|  | Water In Fuel Indicator 1 00b = No 01b = Yes 10b = Error 11b = Not available SPN 97 |  |  |  |  |  |  |  | Not used in FMS-Standard | Fuel Supply Estimated Remaining Distance 1 km/bit 0 offset SPN 8428 |  |  |  |  |  |  |  | Fuel Supply Estimated Remaining Distance 1 km/bit 0 offset SPN 8428 |  |  |  |  |  |  |  | Not used in FMS-Standard |  | Not used in FMS-Standard |  | Not used in FMS-Standard |  | Not used in FMS-Standard |  | Name Name values values values values SPN |
| Description acc. SAE J 1939: Fuel Supply Estimated Remaining Distance: Estimated remaining distance based on existing fuel supply. Water In Fuel Indicator 1: Signal which indicates the presence of water in the fuel. Additional Comment: The mentioned resolution is not related to the accuracy of the signal. The SPN 15268 is used for BEV instead of SPN 8428. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  |  |  |  |  |  |  |  |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  |  |  |  |  |  |  |  |  |  |  |  |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  |  |  |  |  |  |  |  | Page 43 (74) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  |  |  |  |  |  |  |  |  |  |  |  | ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  |  |  |  |  | Issue Version 05 |  |  |  |  | Reg. no. |  |  |
| Subject |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Parameters for Truck FMS-Standard - Cruise Control/Vehicle Speed 1: CCVS1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FEF1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |  |
|  | 65,265 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |  |
|  | 100 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |  |
|  | Data Byte 1 |  |  |  |  |  |  |  | Data Byte 2 |  |  |  |  |  |  |  | Data Byte 3 |  |  |  |  |  |  |  | Data Byte 4 |  |  |  |  |  |  |  | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  |  |  |  |  |  |  |  | Data Byte 8 |  | Byte No |  |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  | 8 | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 |  |  | Bit No |  |
|  | Not used for FMS-Standard |  |  |  |  |  |  |  | Wheel based speed 1/256 km/h Bit gain 0 km/h offset SPN 84 |  |  |  |  |  |  |  | Wheel based speed 1/256 km/h Bit gain 0 km/h offset SPN 84 |  |  |  |  |  |  |  | Clutch switch 00b = pedal released 01b = pedal depressed SPN 598 |  |  |  |  |  |  |  | Not used for FMS-Standard |  | Not used for FMS-Standard |  | Cruise Control States 000b - Off/Disabled 001b – Hold 010b – Accelerate 011b – Decelerate 100b – Resume 101b – Set 110b – Accel. Override 111b - Not available SPN 527 |  |  |  |  |  |  |  |  | Not used for FMS-Standard |  | Name values values values values values values values values SPN |  |
|  | Parking Brake Switch 00b = Parking brake not set 01b = Parking brake set SPN 70 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Brake switch 00b = pedal released 01b = pedal depressed |  |  |  |  |  |  |  |  |  |  |  | PTO state 00000b = off/disabled 00101b = Set 11111b = not available |  |  |  |  |  |  |  |  |  |  | Name values values values |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SPN 597 |  |  |  |  |  |  |  |  |  |  |  | SPN 976 |  |  |  |  |  |  |  |  |  |  | SPN |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Not used for FMS Standard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Cruise control active 00b = switched off 01b = switched on |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name values values values |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SPN 595 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SPN |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE | Scania AB Scania CV Volvo Bus Corporation Renault Trucks | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan | Name of document | FMS-Standard |  | Page | 44 (74) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) | ACEA Task Force HDEI/BCEI |  | Date 11.11.2025 | Approved final | Issue Version 05 | Reg. no. |  |
| Subject |  | FMS-Standard description |  |  |  |  |  |
| Description acc. SAE J 1939: Parking Brake Switch: Switch signal which indicates when the parking brake is set. In general the switch actuated by the operator's park brake control, whether a pedal, lever or other control mechanism. Wheel Based Speed: Speed of the vehicle as calculated from wheel or tailshaft speed. Clutch Switch: Switch signal which indicates that the clutch pedal is being pressed. It is necessary for a safe drivetrain behaviour that the clutch switch is set before the clutch is opened (cruise control function). Brake Switch: Switch signal which indicates that the driver operated brake foot pedal is being pressed. This brake foot pedal is controlling the vehicles' service brake (total vehicle braking application, not park brakes). It is necessary for safe drivetrain behaviour that the switch activates before the physical braking components are activated (i.e. Disengage the cruise control function prior to the activation of friction brakes). Cruise Control Active: Cruise control is switched on. It is not ensured that the engine is controlled by cruise control, as in the case of a large driver's demand the engine is controlled by the driver while cruise control is active (maximum selection of cruise control and driver's demand). The cruise control is set to 0 if a switch off condition occurs. PTO state: This parameter is used to indicate the current state or mode of operation by the power takeoff (PTO) device. It needs to be ensured that each achieved state information be set up to be conveyed in at least one datalink message before a transition to another state is allowed. Off/Disabled 00000b — Used to indicate that the PTO enable switch is in the off position. Set 00101b — Used to indicate that the PTO device is establishing current speed as the operating speed (captured value). Additional comment: The cruise control conditions might vary on different brands. Wheel based speed might vary from tacho speed. The PTO state might be different over the brands (not comparable) due to different internal topology Either SPN 3948 (PTO DE) or SPN 976 (CCVS) is sent. PTO DE message is preferred The clutch switch information is depending on the gear box type * The mentioned resolution is not related to the accuracy of the signal |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  |  |  |  |  |  |  |  |  |  |  |  |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  | Name of document FMS-Standard |  |  |  |  |  |  | Page 45 (74) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  |  |  |  |  | ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  |  | Reg. no. |  |  |
| Subject |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |
| 1.2.2 Electronic Engine Controller #2: EEC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00F003 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |  |
|  | 61,443 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |  |
|  | 50 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |  |
|  | Data Byte 1 | Data Byte 2 |  |  |  |  |  |  |  | Data Byte 3 |  |  |  |  |  |  |  | Data Byte 4 |  | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  |  | Data Byte 8 |  | Byte No |  |
|  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  |  |  |  |  | Bit No |  |
|  | Not used for FMS-Standard | Accelerator pedal position 1 0,4 % / Bit gain 0 % offset SPN 91 |  |  |  |  |  |  |  | Engine Percent Load At Current Speed 1 % / bit, 0 offset 0 to 125 % op. range SPN 92 |  |  |  |  |  |  |  | Not used for FMS-Standard |  | Not used for FMS-Standard |  | Not used for FMS-Standard |  | Not used for FMS-Standard |  |  | Not used for FMS-Standard |  | Name values values values SPN |  |
| Description acc. SAE J 1939: Accelerator Pedal Position: The ratio of actual position of the analogue engine speed/torque request input device (such as an accelerator pedal or throttle lever) to the maximum position of the input device. This parameter is intended for the primary accelerator control in an application. If an application has only one accelerator control, use SPN 91. For on-highway vehicles, this will typically be the operator's accelerator pedal. Although it is used as an input to determine powertrain demand, it also provides anticipatory information to transmission and ASR algorithms about driver actions. Engine Percent Load At Current Speed At Current Speed The ratio of actual engine percent torque (indicated) to maximum indicated torque available at the current engine speed, clipped to zero torque during engine braking. Additional comment: The mentioned resolution is not related to the accuracy of the signal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  |  |  |  |  |  |  |  |  |  |  |  |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  | Name of document FMS-Standard |  |  |  |  |  |  | Page 46 (74) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  |  |  |  |  | ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  |  | Reg. no. |  |  |
| Subject |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |
| 1.2.3 Service Information: SERV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FEC0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |  |
|  | 65,216 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |  |
|  | 1000 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |  |
|  | Data Byte 1 | Data Byte 2 |  |  |  |  |  |  |  | Data Byte 3 |  |  |  |  |  |  |  | Data Byte 4 |  | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  |  | Data Byte 8 |  | Byte No |  |
|  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  |  |  |  |  | Bit No |  |
|  | Not used for FMS-Standard | Service distance 5 km / Bit gain -160 635 km offset SPN 914 |  |  |  |  |  |  |  | Service distance 5 km / Bit gain -160 635 km offset SPN 914 |  |  |  |  |  |  |  | Not used for FMS-Standard |  | Not used for FMS-Standard |  | Not used for FMS-Standard |  | Not used for FMS-Standard |  |  | Not used for FMS-Standard |  | Name values values values SPN |  |
| Description acc. SAE J 1939: Service distance: The distance which can be travelled by the vehicle before the next service inspection is required. A negative distance is transmitted if the service inspection has been passed. The component that requires service is identified by the service component identification (see SPN 911-913, 1379, and 1584) Additional comment: The mentioned resolution is not related to the accuracy of the signal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  | Name of document FMS-Standard |  |  |  |  |  |  |  |  |  |  |  |  | Page 47 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  | ACEA Task Force HDEI/BCEI |  |  |  |  | Date 11.11.2025 |  | Approved final |  |  |  | Issue Version 05 |  |  |  |  |  |  | Reg. no. |  |
| Subject |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.2.4 PTO Drive Engagement: PTODE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FDA4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 64,932 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 100 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 | Data Byte 2 |  | Data Byte 3 | Data Byte 4 | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  |  |  |  |  |  |  |  | Data Byte 8 |  | Byte No |
|  |  |  |  |  |  |  |  |  |  | 8 | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 |  |  | Bit No. |
|  | Not used for FMS-Standard | Not used for FMS-Standard |  | Not used for FMS-Standard | Not used for FMS-Standard | Not used for FMS-Standard |  | Not used for FMS-Standard |  | At least one PTO engaged 00b No PTO drive is engaged 01b At least one PTO drive is engaged 10b Error 11b Not available SPN 3948 |  |  |  |  |  |  |  |  | Not used for FMS-Standard |  | Name Name values values values values values values values values SPN |
|  |  |  |  |  |  |  |  |  |  | Not used for FMS-Standard |  |  |  |  |  |  |  |  |  |  |  |
| Description acc. SAE J 1939: Information relating to the request for engagement, consent for engagement, and status of engagement of various specific physical PTO drives. This message may be broadcast by one or all controllers involved in the enabling of a given PTO drive At least one PTO engaged: Indicates that at least one PTO is engaged Note: This parameter should only be sent by the controller that has knowledge of all PTO drives on the vehicle (e.g, the FMS gateway). Individual PTO drive controllers should broadcast this parameter as "not available". Additional comment: Either SPN 3948 (PTO DE) or SPN 976 (CCVS) is sent. PTO DE message is preferred |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  |  |  |  |  |  |  |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  |  | Page 48 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  | ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  |  | Reg. no. |  |
| Subject |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.2.5 Combination Vehicle Weight: CVW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FE70 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 65,136 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 10 000 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 | Data Byte 2 |  | Data Byte 3 |  |  |  |  |  |  |  | Data Byte 4 |  |  |  |  |  |  |  | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  |  | Data Byte 8 |  | Byte No |
|  |  |  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  |  |  | Bit No. |
|  | Not used for FMS-Standard | Not used for FMS-Standard |  | Gross Combination Vehicle Weight 10 kg/bit 0 offset 0 to 642,550 kg SPN 1760 |  |  |  |  |  |  |  | Gross Combination Vehicle Weight 10 kg/bit 0 offset 0 to 642,550 kg SPN 1760 |  |  |  |  |  |  |  | Not used for FMS-Standard |  | Not used for FMS-Standard |  | Not used for FMS-Standard |  |  | Not used for FMS-Standard |  | Name Name values values values values values SPN |
| Description acc. SAE J 1939: The total weight of the truck and all attached trailers. Additional comment: The mentioned resolution is not related to the accuracy of the signal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  |  |  |  |  |  |  |  |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  |  |  |  |  |  |  |  |  |  |  |  |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  |  |  |  |  |  |  |  | Page 49 (74) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  |  |  |  |  |  |  |  |  |  |  |  | ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  |  |  |  |  | Issue Version 05 |  |  |  |  | Reg. no. |  |  |
| Subject |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Parameters for Bus FMS-Standard - Cruise Control/Vehicle Speed: CCVS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FEF1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |  |
|  | 65,265 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |  |
|  | 100 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |  |
|  | Data Byte 1 |  |  |  |  |  |  |  | Data Byte 2 |  |  |  |  |  |  |  | Data Byte 3 |  |  |  |  |  |  |  | Data Byte 4 |  |  |  |  |  |  |  | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  |  |  |  |  |  |  |  | Data Byte 8 |  | Byte No |  |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  | 8 | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 |  |  | Bit No |  |
|  | Not used in Bus FMS-Standard |  |  |  |  |  |  |  | Wheel based speed 1/256 km/h Bit gain 0 km/h offset SPN 84 |  |  |  |  |  |  |  | Wheel based speed 1/256 km/h Bit gain 0 km/h offset SPN 84 |  |  |  |  |  |  |  | Clutch switch 00b = pedal released 01b = pedal pressed SPN 598 |  |  |  |  |  |  |  | Not used in Bus FMS-Standard |  | Not used in Bus FMS-Standard |  | Cruise Control States 000b - Off/Disabled 001b – Hold 010b – Accelerate 011b – Decelerate 100b – Resume 101b – Set 110b – Accel. Override 111b - Not available SPN 527 |  |  |  |  |  |  |  |  | Not used in Bus FMS-Standard |  | Name values values values values values values values values SPN |  |
|  | Parking Brake Switch 00b = Parking brake not set 01b = Parking brake set SPN 70 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Brake switch 00b = pedal released 01b = pedal depressed SPN 597 |  |  |  |  |  |  |  |  |  |  |  | Not used in Bus FMS-Standard |  |  |  |  |  |  |  |  |  |  | Name values values values values values SPN |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Not used in Bus FMS-Standard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Cruise control active 00b = switched off 01b = switched on SPN 595 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name values values values values SPN |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE | Scania AB Scania CV Volvo Bus Corporation Renault Trucks | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan | Name of document | FMS-Standard |  | Page | 50 (74) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) | ACEA Task Force HDEI/BCEI |  | Date 11.11.2025 | Approved final | Issue Version 05 | Reg. no. |  |
| Subject |  | FMS-Standard description |  |  |  |  |  |
| Description acc. SAE J 1939: Parking Brake Switch: Switch signal which indicates when the parking brake is set. In general the switch actuated by the operator's park brake control, whether a pedal, lever or other control mechanism. Wheel Based Speed: Speed of the vehicle as calculated from wheel or tailshaft speed. Clutch Switch: Switch signal which indicates that the clutch pedal is being pressed. It is necessary for a safe drivetrain behaviour that the clutch switch is set before the clutch is opened (cruise control function). Brake Switch: Switch signal which indicates that the driver operated brake foot pedal is being pressed. This brake foot pedal is controlling the vehicles' service brake (total vehicle braking application, not park brakes). It is necessary for safe drivetrain behaviour that the switch activates before the physical braking components are activated (i.e. Disengage the cruise control function prior to the activation of friction brakes). Cruise Control Active: Cruise control is switched on. It is not ensured that the engine is controlled by cruise control, as in the case of a large driver's demand the engine is controlled by the driver while cruise control is active (maximum selection of cruise control and driver's demand). The cruise control is set to 0 if a switch off condition occurs. Additional comment: The cruise control conditions might vary on different brands. Wheel based speed might vary from tacho speed. The clutch switch information is depending on the gear box type * The mentioned resolution is not related to the accuracy of the signal |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  |  |  |  |  |  |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  | Name of document FMS-Standard |  |  |  |  |  |  | Page 51 (74) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  |  |  |  |  | ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  |  | Reg. no. |  |  |
| Subject |  |  |  |  |  |  |  |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |
| 1.3.2 Electronic Engine Controller #2 : EEC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00F003 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |  |
|  | 61,443 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |  |
|  | 50 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |  |
|  | Data Byte 1 | Data Byte 2 |  |  |  |  |  |  |  | Data Byte 3 | Data Byte 4 |  | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  |  | Data Byte 8 |  | Byte No |  |
|  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  |  |  |  |  |  | Bit No |  |
|  | Not used in Bus FMS-Standard | Accelerator pedal position 0,4 % / Bit gain 0 % offset SPN 91 |  |  |  |  |  |  |  | Not used in Bus FMS-Standard | Not used in Bus FMS-Standard |  | Not used in Bus FMS-Standard |  | Not used in Bus FMS-Standard |  | Not used in Bus FMS-Standard |  |  | Not used in Bus FMS-Standard |  | Name Name values values values values SPN |  |
| Description acc. SAE J 1939: Accelerator Pedal Position: The ratio of actual position of the analogue engine speed/torque request input device (such as an accelerator pedal or throttle lever) to the maximum position of the input device. This parameter is intended for the primary accelerator control in an application. If an application has only one accelerator control, use SPN 91. For on-highway vehicles, this will typically be the operator's accelerator pedal. Although it is used as an input to determine powertrain demand, it also provides anticipatory information to transmission and ASR algorithms about driver actions. Additional comment: The mentioned resolution is not related to the accuracy of the signal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  |  |  |  |  |  |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  | Name of document FMS-Standard |  |  |  |  |  |  | Page 52 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  |  |  |  |  |  |  |  | ACEA Task Force HDEI/BCEI |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  |  | Reg. no. |  |
| Subject |  |  |  |  |  |  |  |  |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |
| 1.3.3 Door Control 1: DC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FE4E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 65,102 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 100 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 |  |  |  |  |  |  |  | Data Byte 2 |  | Data Byte 3 | Data Byte 4 |  | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  |  | Data Byte 8 |  | Byte No |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Bit No. |
|  | Status 2 of doors 00b = all bus doors disabled 01b = at least 1 bus door enabled 10b = error 11b = not available SPN 3411 |  |  |  |  |  |  |  | Not used in Bus-FMS-Standard |  | Not used in Bus-FMS-Standard | Not used in Bus-FMS-Standard |  | Not used in Bus-FMS-Standard |  | Not used in Bus-FMS-Standard |  | Not used in Bus-FMS-Standard |  |  | Not used in Bus-FMS-Standard |  | Name values values values values values values values SPN |
|  | Ramp/Wheel chairlift 00b = inside bus 01b = outside bus 10b = Error 11b = not available SPN 1820 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name values values values values values SPN |
|  | Position of doors 0000b = at least 1 door is open 0001b = closing last door 0010b= all doors closed 0011b-1101b not defined 1110b = Error 1111b = not available SPN 1821 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name values values values values values values values values SPN |
| Description acc. SAE J 1939: Status 2 of doors: Composite indication of all bus door statuses. Enabled means the bus doors are able to be automatically opened or closed.. Ramp/Wheel Chair Lift Position: Signal which indicates the actual position of the ramp / wheel chair lift. Position of Doors: Signal which indicates the actual position of the doors. Additional comment: For the door configuration please contact the manufacturer of the vehicle. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan | Name of document FMS-Standard |  |  | Page 53 (74) |
| --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI | Date 11.11.2025 | Approved final | Issue Version 05 | Reg. no. |
| Subject FMS-Standard description |  |  |  |  |
| 1.3.4 Door Control 2: DC2 Remark: Lock Status: locked -> doors cannot be operated by the driver or a passenger Open Status: closed -> door is completely closed unlocked -> door may be operated by the driver or a passenger open -> door is not completely closed Enable Status: disabled -> door cannot be opened by a passenger enabled -> door can be opened by a passenger Additional comment: For the door configuration please contact the manufacturer of the vehicle. |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  |  |  |  |  |  |  |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  |  |  |  |  |  |  |  |  |  |  |  |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  |  |  |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Page 54 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  |  |  |  |  |  |  |  |  |  |  | ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  |  |  |  |  | Approved final |  |  |  |  |  |  | Issue Version 05 |  |  | Reg. no. |  |
| Subject |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.3.5 Time / Date : TD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FEE6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 65,254 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 1000 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 |  |  |  |  |  |  |  | Data Byte 2 |  |  |  |  |  |  |  | Data Byte 3 |  |  |  |  |  |  |  | Data Byte 4 |  |  |  |  |  |  |  | Data Byte 5 |  |  |  |  |  |  |  | Data Byte 6 |  |  |  |  |  |  |  | Data Byte |  |  | Data Byte |  | Byte No |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  | Bit No. |
|  | Seconds 0.25 s/Bit 0 Offset SPN 959 |  |  |  |  |  |  |  | Minutes 1 min /Bit 0 offset SPN 960 |  |  |  |  |  |  |  | Hours 1 hr/Bit 0 offset SPN 961 |  |  |  |  |  |  |  | Month 1 month/Bit 0 offset SPN 963 |  |  |  |  |  |  |  | Day 0.25 day/Bit 0 offset SPN 962 |  |  |  |  |  |  |  | Year 1 year/Bit 1985 years offset SPN 964 |  |  |  |  |  |  |  | Not used in Bus FMS-Standard |  |  | Not used in Bus FMS-Standard |  | Name values values values values values SPN |
| Description acc. SAE J 1939: Seconds: Part of a parameter used to represent time. Minutes: Part of a parameter used to represent time. Hours: Part of a parameter used to represent time. Month: Part of a parameter used to represent a calendar date. NOTE - A value of 0 for the month is null. The value 1 identifies January; 2 identifies February; etc. Day: Part of a parameter used to represent a calendar date. NOTE - A value of 0 for the date is null. The values 1, 2, 3, and 4 are used to identify the first day of the month; 5, 6, 7, and 8 identify the second day of the month; etc. Year: Part of a parameter used to represent a calendar date. NOTE - A value of 0 for the year identifies the year 1985; a value of 1 identifies 1986; etc. Additional comment: Difference to SAE: broadcast instead of on request. Time base is OEM dependant * |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  |  |  |  |  |  |  |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  | Name of document FMS-Standard |  |  |  |  |  |  | Page 55 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  | ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  |  | Reg. no. |  |
| Subject |  |  |  |  |  |  |  |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |
| 1.3.6 Alternator Speed : AS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FED5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 65,237 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 1000 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 | Data Byte 2 |  | Data Byte 3 |  |  |  |  |  |  |  | Data Byte 4 | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  |  | Data Byte 8 |  | Byte No |
|  |  |  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  |  |  |  | Bit No. |
|  | Not used in Bus FMS-Standard | Not used in Bus FMS-Standard |  | Alternator Status 4 00b = not charging 01b = charging 10b = error 11b = not available SPN 3356 |  |  |  |  |  |  |  | Not used in Bus FMS-Standard | Not used in Bus FMS-Standard |  | Not used in Bus FMS-Standard |  | Not used in Bus FMS-Standard |  |  | Not used in Bus FMS-Standard |  | Name values values values values values SPN |
|  |  |  |  | Alternator Status 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name values values values values values SPN |
|  |  |  |  | 00b = not charging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 01b = charging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 10b = error |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 11b = not available |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | SPN 3355 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Alternator Status 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name values values values values values SPN |
|  |  |  |  | 00b = not charging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 01b = charging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 10b = error |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 11b = not available |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | SPN 3354 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Alternator Status 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name values values values values values SPN |
|  |  |  |  | 00b = not charging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 01b = charging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 10b = error |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 11b = not available |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | SPN 3353 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description acc. SAE J 1939: Actual alternator 1-4 status |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  |  |  |  |  |  |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  |  | Page 56 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  |  |  |  |  |  |  |  | ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  |  | Reg. no. |  |
| Subject |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.3.7 Electronic Transmission Controller 2 : ETC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00F005 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 61,445 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 100 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 |  |  |  |  |  |  |  | Data Byte 2 |  | Data Byte 3 | Data Byte 4 |  |  |  |  |  |  |  | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  |  | Data Byte 8 |  | Byte No |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  |  |  | Bit No. |
|  | Selected Gear 1 gear value/Bit -125 offset negative gear are reverse gears 00000000b = neutral 11111011b = park SPN 524 |  |  |  |  |  |  |  | Not used in Bus FMS-Standard |  | Not used in Bus FMS-Standard | Current Gear 1 gear value/Bit -125 offset negative gear are reverse gears 00000000b = neutral 11111011b = park SPN 523 |  |  |  |  |  |  |  | Not used in Bus FMS-Standard |  | Not used in Bus FMS-Standard |  | Not used in Bus FMS-Standard |  |  | Not used in Bus FMS-Standard |  | Name values values values values values values values SPN |
| Description acc. SAE J 1939: Selected Gear: The gear that the transmission will attempt to achieve during the current shift if a shift is in progress, or the next shift if one is pending (i.e., waiting for torque reduction to initiate the shift). Current Gear : The gear currently engaged in the transmission or the last gear engaged while the transmission is in the process of shifting to the new or selected gear. Transitions toward a destination gear will not be indicated. Once the selected gear has been engaged then Current Gear will reflect that gear. Additional comment: The signal might have limited resolution for gearboxes without electronic control units * |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  | Name of document FMS-Standard |  |  |  |  |  |  | Page 57 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  | ACEA Task Force HDEI/BCEI |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  |  | Reg. no. |  |
| Subject |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |
| 1.3.8 Air Suspension Control 4 : ASC4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FE58 |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 65,112 |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 100 ms |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 | Data Byte 2 |  | Data Byte 3 | Data Byte 4 | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  |  | Data Byte 8 |  | Byte No |
|  | 8 - 1 | 8 - 1 |  | 8 - 1 | 8 - 1 | 8 - 1 |  | 8 - 1 |  | 8- 1 |  |  | 8 - 1 |  | Bit No. |
|  | Bellow Pressure Front Axle Left 0.1 kPa/Bit 0 offset SPN 1725 | Bellow Pressure Front Axle Left 0.1 kPa/Bit 0 offset SPN 1725 |  | Bellow Pressure Front Axle Right 0.1 kPa/Bit 0 offset SPN 1726 | Bellow Pressure Front Axle Right 0.1 kPa/Bit 0 offset SPN 1726 | Bellow Pressure Rear Axle Left 0.1 kPa/Bit 0 offset SPN 1727 |  | Bellow Pressure Rear Axle Left 0.1 kPa/Bit 0 offset SPN 1727 |  | Bellow Pressure Rear Axle Right 0.1 kPa/Bit 0 offset SPN 1728 |  |  | Bellow Pressure Rear Axle Right 0.1 kPa/Bit 0 offset SPN 1728 |  | Name Name values values values values SPN |
| Description acc. SAE J 1939: Used for bellow pressure information Additional comment: The configuration is manufacturer dependant.* The mentioned resolution is not related to the accuracy of the signal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  | Name of document FMS-Standard |  |  |  |  |  |  | Page 58 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  | ACEA Task Force HDEI/BCEI |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  |  | Reg. no. |  |
| Subject |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |
| 1.3.9 Vehicle Electrical Power #4 : VEP4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FCB7 |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 64,695 |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 10000 ms |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 | Data Byte 2 |  | Data Byte 3 | Data Byte 4 | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  |  | Data Byte 8 |  | Byte No |
|  | 8 - 1 | 8 - 1 |  | 8 - 1 | 8 - 1 | 8 - 1 |  | 8 - 1 |  | 8- 1 |  |  | 8 - 1 |  | Bit No. |
|  | Hybrid Battery Pack | Hybrid Battery Pack |  |  |  |  |  |  |  |  |  |  |  |  | Name |
|  | Remaining Charge | Remaining Charge |  |  |  |  |  |  |  |  |  |  |  |  | Name |
|  | 0.0025 %/bit 0 to 160.6375 % | 0.0025 %/bit 0 to 160.6375 % |  | Not used in Bus FMS-Standard | Not used in Bus FMS-Standard | Not used in Bus FMS-Standard |  | Not used in Bus FMS-Standard |  | Not used in Bus FMS-Standard |  |  | Not used in Bus FMS-Standard |  | values values values |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | values |
|  | SPN 5464 | SPN 5464 |  |  |  |  |  |  |  |  |  |  |  |  | SPN |
| Description acc. SAE J 1939: Indicates the hybrid battery pack remaining charge. 0% means no charge remaining, 100% means full charge remaining. Additional comment: The mentioned resolution is not related to the accuracy of the signal. This SPN is used as well for electric vehicles not only for hybrid. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Page 59 (74) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  |  |  |  | Approved final |  |  |  |  |  |  |  | Issue Version 05 |  |  |  |  |  |  |  |  |  | Reg. no. |  |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.3.10 Vehicle Dynamic Stability Control 2 : VDC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00F009 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 61,449 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 100 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 |  |  |  |  |  |  |  | Data Byte 2 |  |  |  |  |  |  |  | Data Byte 3 | Data Byte 4 | Data Byte 5 |  | Data Byte 6 |  |  |  |  |  |  |  | Data Byte 7 |  |  |  |  |  |  |  | Data Byte 8 |  |  |  |  |  |  |  | Byte No |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 - 1 | 8 - 1 | 8 - 1 |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Bit No. |
|  | Steering Wheel Angle 1/1024 rad per bit, - 31.374 rad offset SPN 1807 |  |  |  |  |  |  |  | Steering Wheel Angle 1/1024 rad per bit, - 31.374 rad offset SPN 1807 |  |  |  |  |  |  |  | Not used in Bus FMS-Standard | Not used in Bus FMS-Standard | Not used in Bus FMS-Standard |  | Lateral Acceleration 1/2048 m/s² per bit -15.687 m/s² offset SPN 1809 |  |  |  |  |  |  |  | Lateral Acceleration 1/2048 m/s² per bit -15.687 m/s² offset SPN 1809 |  |  |  |  |  |  |  | Longitudinal Acceleration 0.1 m/s² per bit -12.5 m/s² offset SPN 1810 |  |  |  |  |  |  |  | Name Name values values values values SPN |
| Description acc. SAE J 1939: Steering wheel angle: The main operator`s steering wheel angle (on the steering column, not the actual wheel angle). The vehicle being steered to the left (counterclockwise) results in a positive steering wheel angle. This is the yaw angle of the steering wheel with the z-axis along the centerline of the steering column. This parameter is defined according to a Z-Up axis system and the sign of the value is in accordance to the right-hand rule, as specified in SAE J670. As specified in SAE J670, a Z-Up Axis System has positive X directed forward, positive Y to the left, and positive Z directed up Lateral Acceleration: Indicates a lateral acceleration of the vehicle (the component of vehicle acceleration vector along the Y-axis). A positive lateral acceleration signal results when the vehicle is accelerated to the left. This parameter is defined according to a Z-Up axis system and the sign of the value is in accordance to the right-hand rule, as specified in SAE J670. As specified in SAE J670, a Z-Up Axis System has positive X directed forward, positive Y to the left, and positive Z directed up. See SPN 5347 for an alternate range and resolution. Longitudinal Acceleration: Indicates the longitudinal acceleration of the vehicle. A positive longitudinal acceleration signal results when the vehicle speed increases, regardless of driving the vehicle forward or backward. This parameter is applicable to vehicle speed measurement systems, such as passive wheel speed sensors, that are not capable of determining the direction of the vehicle motion. See SPN 5348 for an alternate longitudinal acceleration. Additional Comment: The mentioned resolution is not related to the accuracy of the signal. The repetition rate can be higher than 100ms and is OEM dependant.* |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  | Page 60 (74) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  | Reg. no. |  |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.3.11 Engine Temperature 1: ET1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FEEE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |  |
|  | 65,262 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |  |
|  | 1000 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |  |
|  | Data Byte 1 |  |  |  |  |  |  |  | Data Byte 2 | Data Byte 3 |  |  |  |  |  |  |  | Data Byte 4 |  |  |  |  |  |  |  | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  | Data Byte 8 |  | Byte No |  |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  |  | Bit No |  |
|  | Engine coolant temperature 1 °C / Bit gain - 40 °C offset SPN 110 |  |  |  |  |  |  |  | Not used for (Bus) FMS-Standard | Engine Oil Temperature 1 0.03125 °C / Bit - 273 °C offset SPN 175 |  |  |  |  |  |  |  | Engine Oil Temperature 1 0.03125 °C / Bit - 273 °C offset SPN 175 |  |  |  |  |  |  |  | Not used for (Bus) FMS-Standard |  | Not used for (Bus) FMS-Standard |  | Not used for (Bus) FMS-Standard |  | Not used for (Bus) FMS-Standard |  | Name values values values SPN |  |
| Description acc. SAE J 1939: Engine Coolant Temperature: Temperature of liquid found in engine cooling system. Engine Oil Temperature: Temperature of the engine lubricant. Additional comment: The mentioned resolution is not related to the accuracy of the signal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  |  | Page 61 (74) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  |  | Approved final |  | Issue Version 05 |  | Reg. no. |  |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.3.12 Ambient Conditions: AMB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FEF5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |  |
|  | 65,269 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |  |
|  | 1000 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |  |
|  | Data Byte 1 | Data Byte 2 |  |  |  |  |  |  |  | Data Byte 3 |  |  |  |  |  |  |  | Data Byte 4 |  |  |  |  |  |  |  | Data Byte 5 |  |  |  |  |  |  |  | Data Byte 6 |  | Data Byte 7 |  | Data Byte 8 |  | Byte No |  |
|  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  | Bit No |  |
|  | Not used for (Bus) FMS-Standard | Cab interior temperature 0.03125 °C / Bit gain - 273 °C offset SPN 170 |  |  |  |  |  |  |  | Cab interior temperature 0.03125 °C / Bit gain - 273 °C offset SPN 170 |  |  |  |  |  |  |  | Ambient Air Temperature 0.03125 °C / Bit gain - 273 °C offset SPN 171 |  |  |  |  |  |  |  | Ambient Air Temperature 0.03125 °C / Bit gain - 273 °C offset SPN 171 |  |  |  |  |  |  |  | Not used for (Bus) FMS-Standard |  | Not used for (Bus) FMS-Standard |  | Not used for (Bus) FMS-Standard |  | Name Name values values values values SPN |  |
| Description acc. SAE J 1939: Ambient Air Temperature: Temperature of air surrounding vehicle. Cab interior temperature: Temperature of air inside the part of the vehicle that encloses the driver and vehicle operating controls. Additional comment: It is not the temperature in the passenger compartment. The mentioned resolution is not related to the accuracy of the signal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  |  |  |  |  |  |  | Page 62 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  |  |  |  |  | Issue Version 05 |  |  |  | Reg. no. |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.3.13 High Voltage Energy Storage System Thermal Management System Status 1 : HVESSTS1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00F109 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 61,705 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 100 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 | Data Byte 2 | Data Byte 3 | Data Byte 4 | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  |  |  |  |  |  |  | Data Byte 8 |  | Byte No |
|  |  |  |  |  |  |  |  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  | Bit No. |
|  | Not used in Bus FMS-Standard | Not used in Bus FMS-Standard | Not used in Bus FMS-Standard | Not used in Bus FMS-Standard | Not used in Bus FMS-Standard |  | Not used in Bus FMS-Standard |  | HVESS Thermal Management System Mode 0000b = Off 0001b = Circulating 0010b = Cooling 0011b = Heating 0100b = Drying 0101b = Reserved 0110b = Reserved 0111b = Reserved 1000b = Reserved 1001b = Reserved 1010b = Reserved 1011b = Reserved 1100b = Reserved 1101b = Reserved 1110b = Faulted 1111b = Not available SPN 8235 |  |  |  |  |  |  |  | Not used in Bus FMS-Standard |  | Name Name Name values values values values values values values values values values values values values values values values SPN |
| Description acc. SAE J 1939: HVESS Thermal Management System Mode: Indicates the operating mode of the Thermal Management System. The heat transfer medium may be (e.g. liquid or air). The cooling system may be fans, liquid coolant, or a combination of mediums. The system may include an air and/or liquid conditioning system with active Cooling, Heating, and/or Drying capabilities. Additional Comment: The mentioned resolution is not related to the accuracy of the signal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Page 63 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  |  |  |  |  |  | Approved final |  |  |  |  |  |  |  | Issue Version 05 |  |  |  |  | Reg. no. |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.3.14 High Voltage Energy Storage System History : HVESSHIST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FC5E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 64,606 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 10000 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 | Data Byte 2 | Data Byte 3 | Data Byte 4 |  |  |  |  |  |  |  | Data Byte 5 |  |  |  |  |  |  |  | Data Byte 6 |  |  |  |  |  |  |  | Data Byte 7 |  |  |  |  |  |  |  | Data Byte 8 |  | Byte No |
|  |  |  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  | Bit No. |
|  | Not used in Bus FMS-Standard | Not used in Bus FMS-Standard | Not used in Bus FMS-Standard | HVESS Total Energy Throughput 100 Watt/bit 0 Watt offset SPN 8211 |  |  |  |  |  |  |  | HVESS Total Energy Throughput 100 Watt/bit 0 Watt offset SPN 8211 |  |  |  |  |  |  |  | HVESS Total Energy Throughput 100 Watt/bit 0 Watt offset SPN 8211 |  |  |  |  |  |  |  | HVESS Total Energy Throughput 100 Watt/bit 0 Watt offset SPN 8211 |  |  |  |  |  |  |  | Not used in Bus FMS-Standard |  | Name Name values values values SPN |
| Description acc. SAE J 1939: HVESS Total Energy Throughput: The total energy throughput which has flowed through High Voltage Energy Storage System (or Pack 1 in systems with only one pack). This parameter is the sum of the absolute values of the Energy into and out of this HVES instance. This parameter provides HVES system level information. If there are multiple packs in the system, then both individual pack messages (e.g., HVESP1, HVESP2 etc.) and system level messages (HVESS) may be required. Additional Comment: This PGN is not sent as a BAM. The mentioned resolution is not related to the accuracy of the signal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  |  |  |  |  |  |  | Page 64 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  |  |  |  |  | Issue Version 05 |  |  |  | Reg. no. |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.3.15 Hybrid System Status 1 : HSS1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FCC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 64,706 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | Every 1 s and on change but no faster than 100 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 | Data Byte 2 |  |  |  |  |  |  |  | Data Byte 3 | Data Byte 4 | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  |  |  |  |  |  |  | Data Byte 8 |  | Byte No |
|  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  | Bit No. |
|  | Not used in Bus FMS-Standard | Propulsion System Active 00b = Propulsion System is not ready to move vehicle if commanded by operator 01b = Propulsion System is ready to move, or is moving the vehicle as commanded by operator 10b = SAE Reserved 11b = Not available SPN 7315 |  |  |  |  |  |  |  | Not used in Bus FMS-Standard | Not used in Bus FMS-Standard | Not used in Bus FMS-Standard |  | Not used in Bus FMS-Standard |  | Not used in Bus FMS-Standard |  |  |  |  |  |  |  | Not used in Bus FMS-Standard |  | Name Name values values values values values values values values values values values values SPN |
| Description acc. SAE J 1939: Propulsion System Active: Indicates when the hybrid system or electric vehicle system is ready to provide or is actively providing propulsion as defined in ARB regulation 13CCR1971.1, which includes the amendments up to the Board hearing date of August 2012. This parameter is called out by the 1971.1 regulation in several areas including, but not limited to: the ignition cycle counter [1971.1 paragraphs (d)(5.5.2)(B) and (d)(5.5.2)(C)], permanent fault code erasure [(d)(2.3.1)(C)(ii)b.3.v], an d denominator incrementing [(d)(4.3.2)(J)(i)] requirements. For example, "for hybrid vehicles, 'ignition cycle' means a driving cycle that begins when the propulsion system active definition is met for at least two seconds plus or minus one second, and ends when the propulsion system active definition is no longer met." NOTE: The parameter name contains the word "active" so that it matches the naming in the ARB regulation. However, this parameter not only indicates that the hybrid system or electric vehicle system is actively aiding or retarding vehicle motion (i.e., generating any positive or negative torque on the driveline); it also shall be asserted when the hybrid system or powertrain system is ready to participate in such activities if vehicle conditions or operator input warrant. Therefore, this parameter is asserted when the vehicle is ready to provide propulsion (e.g., vehicle is ready to be driven, ready to be shifted from "park" to "drive") and doesn't change state again until the system is neither ready to provide, nor actively providing, propulsion. This parameter is required for hybrid vehicles because there is a difference between a conventional vehicle, where powertrain readiness is equivalent to "Engine Running", and a hybrid vehicle, where the engine may or may not be running and yet the vehicle could move. However, if the engine is running (engine speed is nonzero), this parameter shall be set to 1. If the engine speed is zero, then this parameter shall be set based on the hybrid system operating state. For heavy duty vehicles, the hybrid system needs to inform the OBD related components of its propulsion active or readiness state to allow the components to perform OBD related functions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE | Scania AB Scania CV Volvo Bus Corporation Renault Trucks | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan | Name of document | FMS-Standard |  | Page | 65 (74) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) | ACEA Task Force HDEI/BCEI |  | Date 11.11.2025 | Approved final | Issue Version 05 | Reg. no. |  |
| Subject |  | FMS-Standard description |  |  |  |  |  |
| Propulsion System Active cannot be used as an indication of propulsion availability as it is required to indicate "active" whenever the engine in a hybrid system is running. This is regardless of the hybrid system's ability or powertrain system's ability to provide propulsion power. See SPN 8267 for an indication of hybrid system propulsion availability. This message may be used by the Hybrid System Controller to communicate to other OBD related components (e.g engine) the status of the overall hybrid system. It should not be sent by individual hybrid system components (e.g., batteries, inverters, etc.) This message may also be used on electric vehicles to communicate the status of the powertrain system on the electric vehicle. Indicates when the hybrid system or electric vehicle system is ready to provide or is actively providing propulsion. Additional Comment: The mentioned resolution is not related to the accuracy of the signal |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  | Page 66 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  | Approved final |  | Issue Version 05 |  | Reg. no. |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.3.16 Engine Fluid Level/Pressure 1 : EFL/P1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FEEF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 65,263 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 500 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 | Data Byte 2 | Data Byte 3 | Data Byte 4 |  |  |  |  |  |  |  | Data Byte 5 |  | Data Byte 6 |  | Data Byte 7 |  | Data Byte 8 |  | Byte No |
|  |  |  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  |  | Bit No. |
|  | Not used in Bus FMS-Standard | Not used in Bus FMS-Standard | Not used in Bus FMS-Standard | Engine Oil Pressure 1 4 kPa/bit 0 offset SPN 100 |  |  |  |  |  |  |  | Not used in Bus FMS-Standard |  | Not used in Bus FMS-Standard |  | Not used in Bus FMS-Standard |  | Not used in Bus FMS-Standard |  | Name values values SPN |
| Description acc. SAE J 1939: Engine Oil Pressure 1: Gage pressure of oil in engine lubrication system as provided by oil pump. Additional Comment: The mentioned resolution is not related to the accuracy of the signal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  |  |  |  |  |  |  |  |  | Page 67 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  |  |  |  | Date 11.11.2025 |  |  |  |  |  |  | Approved final |  |  |  | Issue Version 05 |  | Reg. no. |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.3.17 Transmission Fluids 1 : TRF1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0x00FEF8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN Hex |
|  | 65,272 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PGN |
|  | 1000 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rep. Rate |
|  | Data Byte 1 | Data Byte 2 | Data Byte 3 | Data Byte 4 | Data Byte 5 |  |  |  |  |  |  |  | Data Byte 6 |  |  |  |  |  |  |  | Data Byte 7 |  | Data Byte 8 |  | Byte No |
|  |  |  |  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  | Bit No. |
|  | Not used in Bus FMS-Standard | Not used in Bus FMS-Standard | Not used in Bus FMS-Standard | Not used in Bus FMS-Standard | Transmission Oil Temperature 1 0.03125 °C/bit -273 °C offset SPN 177 |  |  |  |  |  |  |  | Transmission Oil Temperature 1 0.03125 °C/bit -273 °C offset SPN 177 |  |  |  |  |  |  |  | Not used in Bus FMS-Standard |  | Not used in Bus FMS-Standard |  | Name Name values values SPN |
| Description acc. SAE J 1939: Transmission Oil Temperature 1: First instance of transmission lubricant temperature. Additional Comment: The mentioned resolution is not related to the accuracy of the signal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan | Name of document FMS-Standard |  |  | Page 68 (74) |
| --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI | Date 11.11.2025 | Approved final | Issue Version 05 | Reg. no. |
| Subject FMS-Standard description |  |  |  |  |
| - Examples - Broadcast Announce Message (BAM) for Vehicle ID longer than 8 Byte Transport Protocol – Connection Management (TP.CM) Transport Protocol – Data Transfer (TP.DT) |  |  |  |  |

| 0x00ECFF |  |  |  |  |  |  |  | PGN Hex |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60,671 |  |  |  |  |  |  |  | PGN |
| Byte 1 | Byte 2 | Byte 3 | Byte 4 | Byte 5 | Byte 6 | Byte 7 | Byte 8 | Byte No |
|  |  |  |  |  |  |  |  | Bit No |
| Control byte should be filled with (2016) | Total message size, number of bytes | Total message size, number of bytes | Total number of packets | Reserved should be filled with FF16 | Parameter Group Number of packeted message | Parameter Group Number of packeted message | Parameter Group Number of packeted message | Name values values values |
|  |  |  |  |  |  |  |  | SPN |

| 0x00EBFF |  |  |  |  |  |  |  | PGN Hex |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60,415 |  |  |  |  |  |  |  | PGN |
| Byte 1 | Byte 2 | Byte 3 | Byte 4 | Byte 5 | Byte 6 | Byte 7 | Byte 8 | Byte No |
|  |  |  |  |  |  |  |  | Bit No |
| Sequence Number | Packetized Data | Packetized Data | Packetized Data | Packetized Data | Packetized Data | Packetized Data | Packetized Data | Name values values values SPN |

| Daimler Truck AG Daimler Buses GmbH Volvo Truck Corporation MAN Truck & Bus SE |  | Scania AB Scania CV Volvo Bus Corporation Renault Trucks |  |  |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  |  | Name of document FMS-Standard |  |  |  |  |  |  |  |  |  |  | Page | 69 (74) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) | ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  | Date 11.11.2025 |  |  |  |  | Approved final |  |  | Issue Version 05 |  |  | Reg. no. |  |
| Subject |  |  |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| In the situation shown in Figure 1, a node indicates to the network that it is about to transfer a multipacket message utilizing the service of the transport protocol. In this example, the PGN 00FEEC16 (Vehicle Identification) is being broadcasted to the network. The length of the Vehicle ID in this example is 17. The unused bytes in the last TP.DT are filled with FF16. The originating node first transmits a TP.CM Broadcast Announce Message (BAM) followed by the data packets. No acknowledgment is performed by any of the responders. Originator Responder node node TP.CM_BAM,32,17,3,255,65260 50 to 200 ms TP.DT,1, Vehicle ID Byte 1-7 50 to 200 ms TP.DT,2, Vehicle ID Byte 8-14 50 to 200 ms TP.DT,3, Vehicle ID Byte 15-17, 255, 255, 255,255 Figure 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Time (ms) |  | ID | DLC | Byte 1 |  | Byte 2 | Byte 3 |  | Byte 4 |  |  | Byte 5 |  | Byte 6 | Byte 7 |  | Byte 8 |  |  |  |
|  | 0 |  | PR EC FF SA 16 | 8 | 20 | 16 | 11 16 | 00 16 |  |  | 03 | 16 | FF 16 |  | EC 16 | FE 16 |  | 00 16 |  |  |  |
|  | 50 |  | PR EB FF SA 16 | 8 | 01 | 16 | Vehicle ID byte 1 – 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 100 |  | PR EB FF SA 16 | 8 | 02 | 16 | Vehicle ID byte 8 – 14 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 150 |  | PR EB FF SA 16 | 8 | 03 | 16 | Vehicle ID byte 15 | Vehicle ID byte 16 |  | Vehicle ID byte 17 |  |  | FF 16 |  | FF 16 | FF 16 |  | FF 16 |  |  |  |
| PR is Priority (to be masked) SA is Source Address (to be masked) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB Daimler Buses GmbH Scania CV Volvo Truck Corporation Volvo Bus Corporation MAN Truck & Bus SE Renault Trucks |  |  |  | IVECO Group DAF Trucks N.V. VDL Bus & Coach B.V. Ford Otosan |  | Name of document FMS-Standard |  |  |  |  |  |  |  |  |  |  | Page 70 (74) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) |  | ACEA Task Force HDEI/BCEI |  |  |  | Date 11.11.2025 |  |  | Approved final |  |  |  |  | Issue Version 05 |  |  | Reg. no. |  |  |
| Subject |  |  |  | FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.2 Example SW Identification for buses and/or trucks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | ID |  | Byte 1 | Byte 2 | Byte 3 |  |  | Byte 4 |  | Byte 5 |  |  | Byte 6 | Byte 7 |  | Byte 8 |  |
|  | Supporting Bus-FMS-Standard Version 01 |  | 0x00FD D1 16 |  | X0 16 | 30 16 | 30 | 16 |  | 30 | 16 | 31 | 16 |  | FF 16 | FF 16 |  | FF 16 |  |
|  | Supporting Truck-FMS-Standard Version 02 |  | 0x00FD D1 16 |  | X0 16 | 30 16 | 32 | 16 |  | 30 | 16 | 30 | 16 |  | FF 16 | FF 16 |  | FF 16 |  |
|  | Supporting Bus FMS-Standard Version 03 and Truck FMS-Standard Version 03 |  | 0x00FD D1 16 |  | X0 16 | 30 16 | 33 | 16 |  | 30 | 16 | 33 | 16 |  | FF 16 | FF 16 |  | FF 16 |  |
|  |  | Remark: Byte 2 – Byte 5 are ASCII 3016 = “0” ASCII 3116 = “1” ASCII 3216 = “2” ASCII 3316 = “3” ASCII |  | X=reserved and set to F16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  | Page 71 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  |  | Date 11.11.2025 | Approved final | Issue Version 05 |  | Reg. no. |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.3 Example FMS Tell Tale Status |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ID | Byte 1 | Byte 2 | Byte 3 | Byte 4 | Byte 5 |  | Byte 6 | Byte 7 | Byte 8 |  |  |
|  | 0x00FD7D 16 | B0 16 | FF 16 | FF 16 | 9F 16 | FA 16 |  | A8 16 | 9B 16 | FF 16 |  |  |
|  | TTS | Block ID = 0 Status 1 = Info | Status 2 = not av. Status 3 = not av. | Status 4= not av. Status 5= not av. | Status 6= not av. Status 7= Red | Status 8= Yellow Status 9= not av. |  | Status 10= off Status 11= Yellow | Status 12= Info Status 13= Red | Status 14= not av. Status 15= not av. |  |  |
|  | 0x00FD7D 16 | F1 16 | 8F 16 | AF 16 | B8 16 | FB 16 |  | 88 16 | AA 16 | BB 16 |  |  |
|  | TTS | Block ID = 1 Status 1 = not av. | Status 2 = not av. Status 3 = off | Status 4= not av. Status 5= Yellow | Status 6= off Status 7= Info | Status 8= Info Status 9= not av. |  | Status 10= off Status 11= off | Status 12= Yellow Status 13= Yellow | Status 14= Info Status 15= Info |  |  |
|  | 0x00FD7D 16 | A2 16 | F8 16 | 88 16 | FF 16 | FF 16 |  | AF 16 | FF 16 | FF 16 |  |  |
|  | TTS | Block ID = 2 Status 1 = Yellow | Status 2 = off Status 3 = not av. | Status 4= off Status 5= off | Status 6= not av. Status 7= not av. | Status 8= not av. Status 9= not av. |  | Status 10= not av. Status 11= Yellow | Status 12= not av. Status 13= not av. | Status 14= not av. Status 15= not av. |  |  |
|  | 0x00FD7D 16 | F3 16 | BB 16 | 88 16 | 88 16 | 8B 16 |  | 88 16 | 88 16 | 88 16 |  |  |
|  | TTS | Block ID = 3 Status 1 = not av. | Status 2 = Info Status 3 = Info | Status 4= off Status 5= off | Status 6= off Status 7= off | Status 8= Info Status 9= off |  | Status 10= off Status 11= off | Status 12= off Status 13= off | Status 14= off Status 15= off |  |  |
| Remark: The repetition rate of the PGN is 1000ms which means that the complete "Tell Tale" Msg. (five Blocks) is sent every 5000 ms Due to the repetition rate of the message, it is not guaranteed to include all intermittent signals.* not av. = not available |  |  |  |  |  |  |  |  |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  |  | Page 72 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  | Date 11.11.2025 |  |  | Approved final | Issue Version 05 | Reg. no. |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Block ID | Status No | Status | Name |  | Block ID | Status No |  | Status | Name |  |  |  |  |
|  | 0 | 1 | Info | Cooling air conditioning |  | 2 | 1 |  | Yellow | Steering failure |  |  |  |  |
|  | 0 | 2 | Not av. | High beam, main beam |  | 2 | 2 |  | Off | Height Control (Levelling) |  |  |  |  |
|  | 0 | 3 | Not av. | Low beam, dipped beam |  | 2 | 3 |  | Not av. | Retarder |  |  |  |  |
|  | 0 | 4 | Not av. | Turn signals |  | 2 | 4 |  | Off | Engine Emission system failure (Mil indicator) |  |  |  |  |
|  | 0 | 5 | Not av. | Hazard warning |  | 2 | 5 |  | Off | ESC indication |  |  |  |  |
|  | 0 | 6 | Not av. | Provision for the disabled or handicapped persons |  | 2 | 6 |  | Not av. | Brake lights |  |  |  |  |
|  | 0 | 7 | Red | Parking Brake |  | 2 | 7 |  | Not av. | Articulation |  |  |  |  |
|  | 0 | 8 | Yellow | Brake failure/brake system malfunction |  | 2 | 8 |  | Not av. | Stop Request |  |  |  |  |
|  | 0 | 9 | Not av. | Hatch open |  | 2 | 9 |  | Not av. | Pram request |  |  |  |  |
|  | 0 | 10 | Off | Fuel level |  | 2 | 10 |  | Not av. | Bus stop brake |  |  |  |  |
|  | 0 | 11 | Yellow | Engine coolant temperature |  | 2 | 11 |  | Yellow | AdBlue level |  |  |  |  |
|  | 0 | 12 | Info | Battery charging condition |  | 2 | 12 |  | Not av. | Raising |  |  |  |  |
|  | 0 | 13 | Red | Engine oil |  | 2 | 13 |  | Not av. | Lowering |  |  |  |  |
|  | 0 | 14 | Not av. | Position lights,side lights |  | 2 | 14 |  | Not av. | Kneeling |  |  |  |  |
|  | 0 | 15 | Not av. | Front fog light |  | 2 | 15 |  | Not av. | Engine compartment temperature |  |  |  |  |
|  | 1 | 1 | Not av. | Rear fog light |  | 3 | 1 |  | Not av. | Auxillary air pressure |  |  |  |  |
|  | 1 | 2 | Not av. | Park Heating |  | 3 | 2 |  | Info | Air filter clogged |  |  |  |  |
|  | 1 | 3 | Off | Engine |  | 3 | 3 |  | Info | Fuel filter differential pressure |  |  |  |  |
|  | 1 | 4 | Not av. | Service, call for maintenance |  | 3 | 4 |  | Off | Seat belt |  |  |  |  |
|  | 1 | 5 | Yellow | Transmission fluid temperature |  | 3 | 5 |  | Off | EBS |  |  |  |  |
|  | 1 | 6 | Off | Transmission failure/malfunction |  | 3 | 6 |  | Off | Lane departure indication |  |  |  |  |
|  | 1 | 7 | Info | Anti-lock brake system failure |  | 3 | 7 |  | Off | Advanced emergency braking system |  |  |  |  |
|  | 1 | 8 | Info | Worn brake linings |  | 3 | 8 |  | Info | ACC |  |  |  |  |
|  | 1 | 9 | Not av. | Windscreen washer fluid/windshield washer fluid |  | 3 | 9 |  | Off | Trailer connected |  |  |  |  |
|  | 1 | 10 | Off | Tire failure/malfunction |  | 3 | 10 |  | Off | ABS Trailer 1,2 |  |  |  |  |
|  | 1 | 11 | Off | Malfunction/general failure |  | 3 | 11 |  | Off | Airbag |  |  |  |  |
|  | 1 | 12 | Yellow | Engine oil temperature |  | 3 | 12 |  | Off | EBS Trailer 1,2 |  |  |  |  |
|  | 1 | 13 | Yellow | Engine oil level |  | 3 | 13 |  | Off | Tachograph indication |  |  |  |  |
|  | 1 | 14 | Info | Engine coolant level |  | 3 | 14 |  | Off | ESC switched off |  |  |  |  |
|  | 1 | 15 | Info | Steering fluid level |  | 3 | 15 |  | Off | Lane departure warning switched off |  |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  | Page 73 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  | Date 11.11.2025 | Approved final |  | Issue Version 05 | Reg. no. |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 Overview Messages |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | (signal) name | Mandatory | rep. rate | remarks / comments |  |  |  |  |  |  |
|  | page | PGN | SPN | e.g. milage, fuel consumption | Truck only | in ms |  |  |  |  |  |  |  |
|  | Truck AND Bus Section Truck AND Bus Section Truck AND Bus Section Truck AND Bus Section Truck AND Bus Section Truck AND Bus Section Truck AND Bus Section |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 11 | 65257 | 250 | Engine total fuel used |  | 1000 | 4 bytes, 0 to +2 105 540 607,5 L |  |  | Might be set to “not available” if SPN 5054 is available |  |  |  |
|  | 12 | 65276 | 96 | Fuel level 1 | X (worldwide) | 1000 | 1 Byte |  |  | - |  |  |  |
|  | 12 | 65276 | 38 | Fuel level 2 |  | 1000 | 1 Byte |  |  | Only used if different fuel types in different storage container |  |  |  |
|  | 13 | 61444 | 513 | Actual Engine – Percent Torque | X (worldwide) | 20 | 1 % / Bit, -125 % offset |  |  | - |  |  |  |
|  | 13 | 61444 | 190 | Engine speed | X (worldwide) | 20 | 2 Byte, 0-8031,875 rpm |  |  | - |  |  |  |
|  | 14 | 65253 | 247 | Engine total hours of Operation | X (worldwide) | 1000 | 4 bytes, 0 to 210 554 060,75 h |  |  | Counter is Engine running dependant |  |  |  |
|  | 15 | 65260 | 237 | vehicle identification number | X (worldwide) | 10000 | variable, max 200 char. |  |  | Will be sent every 10 sec |  |  |  |
|  | 16 | 64977 | 2806 | SW-version supported | X (worldwide) | 10000 | Indicator for SW version supported |  |  | - |  |  |  |
|  | 16 | 64977 | 2804 | Diagnostics supported | X (worldwide) | 10000 | indicator for diagnosis session support |  |  | - |  |  |  |
|  | 16 | 64977 | 2805 | Requests supported | X (worldwide) | 10000 | indicator for request supported |  |  | - |  |  |  |
|  | 18 | 65217 | 917 | High resolution total vehicle distance | X (worldwide) | 1000 | 4 bytes, 0 - 21 055 406 km; without TCO |  |  | Resolution may be not within the SAE values |  |  |  |
|  | 19 | 65132 | 1611 | Vehicle motion | X (EU) | 20/50 | With digital tachograph |  |  | rep. rate tacho dependant |  |  |  |
|  | 19 | 65132 | 1613 | Driver 2 working state | X (EU) | 20/50 | With digital tachograph |  |  | rep. rate tacho dependant |  |  |  |
|  | 19 | 65132 | 1612 | Driver 1 working state | X (EU) | 20/50 | With digital tachograph |  |  | rep. rate tacho dependant |  |  |  |
|  | 19 | 65132 | 1614 | Vehicle overspeed |  | 20/50 | With digital tachograph |  |  | rep. rate tacho dependant |  |  |  |
|  | 19 | 65132 | 1617 | Driver 1 time rel. states |  | 20/50 | With digital tachograph |  |  | rep. rate tacho dependant |  |  |  |
|  | 19 | 65132 | 1618 | Driver 2 time rel. states |  | 20/50 | With digital tachograph |  |  | rep. rate tacho dependant |  |  |  |
|  | 19 | 65132 | 1615 | Driver 1 card | X (EU) | 20/50 | With digital tachograph |  |  | rep. rate tacho dependant |  |  |  |
|  | 19 | 65132 | 1616 | Driver 2 card | X (EU) | 20/50 | With digital tachograph |  |  | rep. rate tacho dependant |  |  |  |
|  | 19 | 65132 | 1619 | Direction indicator |  | 20/50 | With digital tachograph |  |  | rep. rate / availability is tachograph dependant. |  |  |  |
|  | 19 | 65132 | 1620 | Tachograph performance | X (EU) | 20/50 | With digital tachograph |  |  | rep. rate tacho dependant |  |  |  |
|  | 19 | 65132 | 1621 | Handling information | X (EU) | 20/50 | With digital tachograph |  |  | rep. rate tacho dependant |  |  |  |
|  | 19 | 65132 | 1622 | System event | X (EU) | 20/50 | With digital tachograph |  |  | rep. rate tacho dependant |  |  |  |
|  | 19 | 65132 | 1624 | Tachograph vehicle speed | X (EU) | 20/50 | With digital tachograph - 2 bytes |  |  | rep. rate tacho dependant/might differ from the wheel based speed |  |  |  |
|  | 21 | 65262 | 110 | Engine coolant temperature | X (worldwide) | 1000 | -40° to 210° |  |  | - |  |  |  |
|  | 22 | 65269 | 171 | Ambient Air Temperature | X (worldwide) | 1000 | 0.03125 °C / Bit gain |  |  | - 273 °C offset |  |  |  |
|  | 23 | 65131 | 1625/1626 | Driver 1 / Driver 2 Identification | X (EU) | 10000 | If a driver ID is available the message is sent with a Broadcast Announce Message (BAM) |  |  | Diff. to SAE: broadcast instead of on request |  |  |  |
|  | 24 | 65266 | 183 | Fuel rate | X (worldwide) | 100 | 0.05 L/h per bit, 0 to 3,212.75 L/h |  |  | Calculated values given as indications, not as contractual |  |  |  |
|  | 24 | 65266 | 184 | Instantaneous Fuel Economy | X (worldwide) | 100 | 1/512 km/L per bit, 0 to 125,5 km/L |  |  | Calculated values given as indications, not as contractual |  |  |  |
|  | 25 | 65198 | 1087 | Service Brake Air Pressure Circuit #1 | X (worldwide) | 1000 | 8 kPa/Bit, 0 offset |  |  | - |  |  |  |
|  | 25 | 65198 | 1088 | Service Brake Air Pressure Circuit #2 | X (worldwide) | 1000 | 8 kPa/Bit, 0 offset |  |  | - |  |  |  |
|  | 26 | 64777 | 5054 | High resolution engine total fuel used |  | 1000 | 0.001 L/bit, 0 to 4,211,081.215 L |  |  | Is implemented if technical possible |  |  |  |
|  | 27 | 65110 | 1761 | Aftertreatment 1 Diesel Exhaust Fluid Tank 1 Level |  | 1000 | 0.4 %/bit, 0 % offset |  |  |  |  |  |  |
|  | 28 | 64893 |  | FMS Tell Tale Status | X (worldwide ,EU) Not all tell tales | 1000 | - |  |  | 5 blocks => Rep.rate for each tell tale status might be 5 sec if all blocks are sent |  |  |  |
|  | 32 | 61441 | 521 | Brake Pedal Position |  | 100 | - |  |  | - |  |  |  |
|  | 33 | 64962 | 5837 | Fuel Type |  | 10000 | - |  |  | - |  |  |  |
|  | 36 | 65199 | 1040 | Total fuel used (Gaseous) |  | 1000 | - |  |  | - |  |  |  |
|  | 37 | 61440 | 900 | Retarder Torque Mode |  | 100 | 16 states/4 bit, 0 offset |  |  | - |  |  |  |
|  | 37 | 61440 | 520 | Actual Retarder - Percent Torque |  | 100 | 1 %/bit, -125 % offset |  |  | - |  |  |  |
|  | 37 | 61440 | 1716 | Retarder Selection, non-engine |  | 100 | 0.4 %/bit, 0 % offset |  |  | The value is related to the drive line retarder |  |  |  |
|  | 38 | 65258 | 928 | Axle location |  | 1000 | - |  |  | If info of more axles available it will be updated with each repetition |  |  |  |
|  | 38 | 65258 | 928 | Tire location |  | 1000 | - |  |  | - |  |  |  |
|  | 38 | 65258 | 582 | Axle weight |  | 1000 | - |  |  | - |  |  |  |
|  | 39 | 64184 | 13171 | DC Charging State |  | 1000 | - |  |  | - |  |  |  |
|  | 40 | 64617 | 15268 | HVESS Estimated Remaining Distance |  | 10000 | - |  |  | - |  |  |  |
|  | 41 | 65268 | 929 | Tire Location |  | 10000 | - |  |  | - |  |  |  |
|  | 41 | 65268 | 241 | Tire Pressure |  | 10000 | - |  |  | - |  |  |  |
|  | 42 | 65279 | 97 | Water In Fuel Indicator 1 |  | 10000 | - |  |  | - |  |  |  |
|  | 42 | 65279 | 8428 | Fuel Supply Estimated Remaining Distance |  | 10000 | - |  |  | SPN 15268 is used for BEV instead of SPN 8428 |  |  |  |

| Daimler Truck AG Scania AB IVECO Group Daimler Buses GmbH Scania CV DAF Trucks N.V. Volvo Truck Corporation Volvo Bus Corporation VDL Bus & Coach B.V. MAN Truck & Bus SE Renault Trucks Ford Otosan |  |  |  |  |  |  |  | Name of document FMS-Standard |  |  |  | Page 74 (74) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Issuer (dept., name, phone, sign) ACEA Task Force HDEI/BCEI |  |  |  |  |  |  |  | Date 11.11.2025 | Approved final |  | Issue Version 05 | Reg. no. |  |
| Subject FMS-Standard description |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Truck only Section Truck only Section Truck only Section Truck only Section Truck only Section Truck only Section Truck only Section |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 43 | 65265 | 84 | Wheel based speed |  | 100 | may differ from TCO1 |  |  | - |  |  |  |
|  | 43 | 65265 | 598 | Clutch switch |  | 100 | two bit status |  |  | in trucks with automatic gear => send as not available |  |  |  |
|  | 43 | 65265 | 597 | Brake switch |  | 100 | two bit status |  |  | - |  |  |  |
|  | 43 | 65265 | 595 | Cruise control active |  | 100 | two bit status |  |  | in trucks with no cruise control => send as not available |  |  |  |
|  | 43 | 65265 | 976 | PTO state |  | 100 | Either SPN 3948 (PTODE) or SPN 976 is sent |  |  | SPN 3948 (PTO DE) message is preferred |  |  |  |
|  | 43 | 65265 | 70 | Parking brake switch |  | 100 |  | - |  |  | - |  |  |
|  | 43 | 65265 | 527 | Cruise Control States |  | 100 |  | - |  |  | - |  |  |
|  | 45 | 61443 | 91 | Accelerator pedal position 1 | X (worldwide) | 50 | 1 Byte |  |  | - |  |  |  |
|  | 45 | 61443 | 92 | Engine Percent Load At Current Speed | X (worldwide) | 50 | 1 % / bit, 0 to 125 % operational range |  |  | - |  |  |  |
|  | 46 | 65216 | 914 | Service distance |  | 1000 | - |  |  | - |  |  |  |
|  | 47 | 64932 | 3948 | At least one PTO engaged |  | 100 | Either SPN 3948 or SPN 976 (CCVS) is sent |  |  | SPN 3948 (PTO DE) message is preferred |  |  |  |
|  | 48 | 65136 | 1760 | Gross Combination Vehicle Weight |  | 10000 | 0 to 642,550 kg |  |  | Diff. to SAE: broadcast instead of on request |  |  |  |
|  | Bus only Section Bus only Section Bus only Section Bus only Section Bus only Section Bus only Section Bus only Section |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 49 | 65265 | 84 | Wheel based speed |  | 100 | may differ from TCO1 |  |  | - |  |  |  |
|  | 49 | 65265 | 598 | Clutch switch |  | 100 | two bit status |  |  | in trucks with automatic gear => send as not available |  |  |  |
|  | 49 | 65265 | 597 | Brake switch |  | 100 | two bit status |  |  | - |  |  |  |
|  | 49 | 65265 | 595 | Cruise control active |  | 100 | two bit status |  |  | in trucks with no cruise control => send as not available |  |  |  |
|  | 49 | 65265 | 70 | Parking Brake Switch |  | 100 | - |  |  | - |  |  |  |
|  | 49 | 65265 | 527 | Cruise Control States |  | 100 | - |  |  | - |  |  |  |
|  | 51 | 61443 | 91 | Accelerator pedal position 1 |  | 50 | 1 Byte |  |  | - |  |  |  |
|  | 52 | 65102 | 3411 | Status 2 of doors |  | 100 | - |  |  | - |  |  |  |
|  | 52 | 65102 | 1820 | Ramp/wheel chairlift status |  | 100 | - |  |  | - |  |  |  |
|  | 52 | 65102 | 1821 | Position of doors |  | 100 | - |  |  | For door configuration please contact the vehicle manufacturer |  |  |  |
|  | 53 | 64933 | 3412-3441 | Status of doors 1 - 10 |  | 100 | - |  |  | For door configuration please contact the vehicle manufacturer |  |  |  |
|  | 54 | 65254 | 959 | Seconds |  | 1000 | - |  |  | Difference to SAE: broadcast instead of on request |  |  |  |
|  | 54 | 65254 | 960 | Minutes |  | 1000 | - |  |  | Difference to SAE: broadcast instead of on request |  |  |  |
|  | 54 | 65254 | 961 | Hours |  | 1000 | - |  |  | Difference to SAE: broadcast instead of on request |  |  |  |
|  | 54 | 65254 | 963 | Month |  | 1000 | - |  |  | Difference to SAE: broadcast instead of on request |  |  |  |
|  | 54 | 65254 | 962 | Day |  | 1000 | - |  |  | Difference to SAE: broadcast instead of on request |  |  |  |
|  | 54 | 65254 | 964 | Year |  | 1000 | - |  |  | Difference to SAE: broadcast instead of on request |  |  |  |
|  | 55 | 65237 | 3356 | Alternator Status 4 |  | 1000 | - |  |  | - |  |  |  |
|  | 55 | 65237 | 3355 | Alternator Status 3 |  | 1000 | - |  |  | - |  |  |  |
|  | 55 | 65237 | 3354 | Alternator Status 2 |  | 1000 | - |  |  | - |  |  |  |
|  | 55 | 65237 | 3353 | Alternator Status 1 |  | 1000 | - |  |  | - |  |  |  |
|  | 56 | 61445 | 524 | Selected Gear |  | 100 | - |  |  | - |  |  |  |
|  | 56 | 61445 | 523 | Current Gear |  | 100 | - |  |  | - |  |  |  |
|  | 57 | 65112 | 1725 | Bellow Pressure Front Axle Left |  | 100 | - |  |  | - |  |  |  |
|  | 57 | 65112 | 1726 | Bellow Pressure Front Axle Right |  | 100 | - |  |  | - |  |  |  |
|  | 57 | 65112 | 1727 | Bellow Pressure Rear Axle Left |  | 100 | - |  |  | - |  |  |  |
|  | 57 | 65112 | 1728 | Bellow Pressure Rear Axle Right |  | 100 | - |  |  | - |  |  |  |
|  | 58 | 64695 | 5464 | Hybrid Battery Pack Remaining Charge |  | 10000 | - |  |  | This SPN is used as well for electric vehicles not only for hybrid. |  |  |  |
|  | 59 | 61449 | 1807 | Steering wheel angle |  | 100 | - |  |  | - |  |  |  |
|  | 59 | 61449 | 1809 | Lateral Acceleration |  | 100 | - |  |  | - |  |  |  |
|  | 59 | 61449 | 1810 | Longitudinal Acceleration |  | 100 | - |  |  | - |  |  |  |
|  | 60 | 65262 | 110 | Engine coolant temperature |  | 1000 | - |  |  | - |  |  |  |
|  | 60 | 65262 | 175 | Engine Oil Temperature 1 |  | 1000 | - |  |  | - |  |  |  |
|  | 61 | 65269 | 170 | Cab interior temperature |  | 1000 | - |  |  | It is not the temperature in the passenger compartment. |  |  |  |
|  | 61 | 65269 | 171 | Ambient Air Temperature |  | 1000 | - |  |  | - |  |  |  |
|  | 62 | 61705 | 8235 | HVESS Thermal Management System Mode |  | 100 | - |  |  | - |  |  |  |
|  | 63 | 64606 | 8211 | HVESS Total Energy Throughput |  | 10000 | - |  |  | - |  |  |  |
|  | 64 | 64706 | 7315 | Propulsion System Active |  | 100 | - |  |  | - |  |  |  |
|  | 66 | 65263 | 100 | Engine Oil Pressure 1 |  | 500 | - |  |  | - |  |  |  |
|  | 67 | 65272 | 177 | Transmission Oil Temperature 1 |  | 1000 | - |  |  | - |  |  |  |